NEEDED 2010 FLORIDA BUILDING CODE FIXES - OTHER: RULE 61G20 - 1.001

|  | TEXT | RATIONALE |
| --- | --- | --- |
|  | Florida Building Code, Building |  |
|  | Chapter 1 - Scope and Administration Add Item (k) 102.2 Building. The provisions of the Florida Building Code shall apply to the construction, erection, alteration, modification, repair, equipment, use and occupancy, location, maintenance, removal and demolition of every public and private building, structure or facility or floating residential structure, or any appurtenances connected or attached to such buildings, structures or facilities. Additions, alterations, repairs and changes of use or occupancy group in all buildings and structures shall comply with the provisions provided in Chapter 34 of this code. The following buildings, structures and facilities are exempt from the Florida Building Code as provided by law, and any further exemptions shall be as determined by the legislature and provided by law: (k) A building or structure having less than 1,000 square feet which is constructed and owned by a natural person for hunting and which is repaired or reconstructed to the same dimension and condition as existed on January 1, 2011, if the building or structure: 1. Is not rented or leased or used as a principal residence; 2. Is not located within the 100-year floodplain according to the Federal Emergency Management Agency’s current Flood Insurance Rate Map; and 3. Is not connected to an off-site electric power or water supply. | Change to State law. HB704 |
|  | Staff Comment - Glitch |  |
|  | Chapter 2 – Definitions |  |
|  | Add definition of “Bedroom” as follows: (2) DEFINITIONS.—as used in ss. 381.0065-381.0067, the term: (b)1. “Bedroom” means a room that can be used for sleeping and that: a. For site-built dwellings has a minimum of 70 square feet of conditioned space; b. For manufactured homes is constructed according to the standards of the United States Department of Housing and Urban Development and has a minimum of 50 square feet of floor area; c. Is located along an exterior wall; d. Has a closet and a door or an entrance where a door could be reasonably installed; and e. Has an emergency means of escape and rescue opening to the outside in accordance with the Florida Building Code. | Change to State law. HB704 |
|  | Staff Comment - Glitch |  |
|  | Chapter 4 - Special Detailed Requirements Based on Use and Occupancy |  |
|  | TABLE 424.1.6.1 PUBLIC SWIMMING POOL—REQUIRED FIXTURE COUNT SIZE OF POOL; MEN'S RESTROOM; WOMEN'S RESTROOM. Urinals; WC; Lavatory; WC; Lavatory. 0 - 2,500 sq. ft.; 1; 1; 1; 1; 1. 2,501 - 5,000 sq. ft.; 2; 1; 1; 5; 1. 5,001 - 7,500 sq. ft.; 2; 2; 2; 6; 2. 7,501 - 10,000 sq. ft.; 3; 2; 3; 8; 3. For SI: 1 square foot = 0.0929 m2.. | Conflicts within the updated code. Editorial – correct format. |
|  | Staff Comment - Glitch |  |
|  | Changes requested by DOH for Consistency with Rule 64E-9 Cartridge filter sizing is 0.075 in FBC and 0.375 gpm/sq ft in 64E-9, FAC. 424.1.6.5.5.1 Filter capacities. The maximum filtration rate in gallons per minute per square foot of filter area shall be: 15 [20 if so approved using the procedure stated in Section 424.1.6.5.1 for high rate sand filters, 3 for rapid sand filters, 0.075 0.375 for pleated cartridge filters and 2 for Diatomaceous Earth (D.E.) type filters]. | The proposed code change is within glitch criteria permitted for inconsistencies with federal regulations. Editorial – correct filter capacities as per DOH’s rule. |
|  | Staff Comment - Glitch |  |
|  | Changes requested by DOH for Consistency with Rule 64E-9. Turnover time for a water slide plunge pool is 3 hours in FBC 424.1 and 2 hours in 64E-9, FAC. 424.1.9.2.6.1 Recirculation rate. The recirculation-filtration system of water slides shall recirculate and filter a water volume equal to the total water volume of the facility in a period of 32 hours or less. | The proposed code change is within glitch criteria permitted for inconsistencies with federal regulations. |
|  | Staff Comment - Glitch |  |
|  | Changes requested by DOH for Consistency with Rule 64E-9. Very small spas are allowed to be constructed without pH adjustment feeders in 424.1.6.5.16.3, but not in 64E-9007(16)(c), FAC. 424.1.6.5.16.3 Feeders for PH adjustment. Feeders for PH adjustment shall be provided on all pools., except spa pools of less than 100 square feet (9 m2) of pool water surface area and pools utilizing erosion type chlorinators feeding chlorinated isocyanurates. PH adjustment feeders shall be positive displacement type, shall be adjustable from 0 to full range, and shall have an electrical interlock with the circulation pump to prevent discharge when the recirculation pump is not operating. When soda ash is used for PH adjustment, the maximum concentration of soda ash solution to be fed shall not exceed 1/2-pound (.2 kg) soda ash per gallon of water. Feeders for soda ash shall be capable of feeding a minimum of 3 gallons (11 L) of the above soda ash solution per pound of gas chlorination capacity. The minimum size of the solution reservoirs shall not be less than 50 percent of the maximum daily capacity of the feeder. The solution reservoirs shall be marked to indicate the type of contents. | The proposed code change is within glitch criteria permitted for inconsistencies with federal regulations. |
|  | Staff Comment - Glitch |  |
|  | Chapter 7 - Fire and Smoke Protection Features |  |
|  | 709.1 General. The following wall assemblies shall comply with this section. 1. Walls separating dwelling units in the same building as required by Section 4389.2. 2. Walls separating sleeping units in the same building as required by Section 4389.2. | Conflicts within the updated code. Editorial – correct reference section. |
|  | Staff Comment - Glitch |  |
|  | Chapter 15 - Roof Assemblies and Rooftop Structures |  |
| 2 | 1501.1 Scope. The provisions of this chapter shall govern the design, materials, construction and quality of roof assemblies, and rooftop structures. Exception: Buildings and structures located within the high-velocity hurricane zone shall comply with the provisions of Section 1503.6 7 and Sections 1512 through 1525. | Conflicts within the updated code. Editorial – correct reference section. |
|  | Staff Comment - Glitch |  |
|  | Chapter 35 - Referenced Standards |  |
|  | 99 – 02 Standard for Health Care Facilities…. 99 – 05 Standard for Health Care Facilities… | Conflicts within the updated code. Editorial – remove absolute standard. |
|  | Staff Comment - Glitch |  |
|  | Florida Building Code, Plumbing |  |
|  | Table 603.1 of the 2010 Florida Building Code, Plumbing. The first column “No. of Fixture Units Flush Tanks WC”, 3rd raw – the numbers should be as follows: 56 – 5885 | Conflicts within the updated code. Editorial – miss print. |
|  | Staff Comment - Glitch |  |