Accessibility Technical Advisory Committee – Errata/Glitch

8th Edition (2023) Florida Building Code, Building

CHAPTER 10 MEANS OF EGRESS

F/AC-FBC- B-Ch. 10 – Errata #1

Katrina Fumagali, AIA

Mr. Madani,

Thank you for taking the time to speak with me today.

Below is a summary of our discussion:

The Florida Accessibility Code (FAC) is a standalone guideline that has been updated.

By law, if there is a conflict between the Florida Building Code and the Florida Accessibility Code, we shall always defer to the FAC.

These are the references from the FBC:

101.4.8 Accessibility.

For provisions related to accessibility, refer to the Florida Building Code, Accessibility.

# 1009.1 Accessible means of egress required.

Accessible means of egress shall be provided in accordance with the applicable sections of the Florida Building Code, Accessibility.

Regarding the conflict when it comes to turnstiles:

Per the FAC 404.2.1, turnstiles shall not be part of an accessible route.

FBC Accessibility 404 DOORS, DOORWAYS, AND GATES

404.2.1 Revolving Doors, Gates, and Turnstiles.

Revolving doors, revolving gates, and turnstiles shall not be part of an accessible route.

VS

FBC, Building, 7th Edition 1010 DOORS, GATES AND TURNSTILES

1010.3.1.1 Clear width.

Where located as part of an accessible route, turnstiles shall have not less than 36 inches clear at and below a height of 34 inches, not less than 32 inches clear width between 34 inches and 80 inches and shall consist of a mechanism other than a revolving device.

8th Edition (2023) FBC – Building

1010.5.1.1 Clear width. Where located as part of an accessible route, turnstiles shall have not less than 36 inches (914 mm) clear at and below a height of 34 inches (864 mm), not less than 32 inches (813 mm) clear width between 34 inches (864 mm) and 80 inches (2032 mm) and shall consist of a mechanism other than a revolving device.

Fix –

1010.5.1.1 Clear width. Reserved Where located as part of an accessible route, turnstiles shall have not less than 36 inches (914 mm) clear at and below a height of 34 inches (864 mm), not less than 32 inches (813 mm) clear width between 34 inches (864 mm) and 80 inches (2032 mm) and shall consist of a mechanism other than a revolving device.

TAC Recommendation:

Commission Action:

Comment –

BOAF Building Officials Association of Florida CDC Code Development Committee

Proposal:		Turnstiles conflict between FAC FBCB

FAC 404.2.1 turnstiles shall not be part of accessible route

Fix FBCB 8th Ed 1010.5.1.1 Clear width (reserved)

Comment:		Conflict should be corrected

Approve/Oppose: 	Approve

8th Edition (2023) Florida Building Code, Plumbing

CHAPTER 4 FIXTURES, FAUCETS AND FIXTURE FITTINGS

P-FBC-P – Ch. 4 – Errata #1

Staff

Correlation with the Florida Accessibility Code -

403.1.1 Fixture calculations.

Exceptions:

---

2. Where multi-user facilities are designed to serve all genders, the minimum fixture count shall be calculated 100 percent, based on total occupant load. In such multi-user user facilities, each fixture type shall be in accordance with ICC A117.1 Florida Building Code, Accessibility, and each urinal that is provided shall be located in a stall.

TAC Recommendation:

Commission Action:

Comment –

BOAF Building Officials Association of Florida CDC Code Development Committee

Proposal:		Correlation with FAC Replace ICC A117.1 with FAC

Comment:		Needed correction

Approve/Oppose:	Approve