Manufactured (Modular) Building Program

NEWLY MANUFACTURED (MODULAR) BUILDINGS:

- What type of building is regulated by the Department of Business and Professional Regulation (DBPR)  Manufactured (Modular) Buildings Program?

As defined in Rule 61-41, F.A.C., a “closed construction” structure, building assembly or system of subassemblies, ………manufactured for installation or erection…… as a finished building or as part of a finished building………”.  “Closed construction is defined as a building, component, assembly, etc……manufactured in such a manner that all portions cannot be readily inspected at the installation site without disassembly or destruction thereof”.  This includes residential, commercial, storage sheds, portable classrooms, guard shacks, communication buildings, kiosks, gazebos, ice vending buildings and ATMs (if occupiable).

NOTE: Mobile X-Ray/MRI Units, bathrooms, cafeterias, etc. that are mounted on a trailer do not come under the jurisdiction of the FBC or DBPR because they are not considered a building.

- Are manufactured buildings bearing a DBPR insignia required to comply with the Florida Building Code (FBC)?

Yes. Manufactured (modular) buildings constructed on or after March 1, 2002, are required to comply with all technical and applicable administrative sections of the FBC and all subsequent revisions and supplements in effect at the time of construction. Buildings built prior to March 01, 2002, must comply with the applicable code in effect at the time of construction or renovation.  All site related work (erection, assembly and construction at the site, plus all foundation, utility connection, etc,) are subject to local permitting and inspections. (Chapter 1, FBC and Chapter  553.80(1),(d) FS).

Exception: Construction Trailers and Temporary Offices are not required to comply with the FBC—(see # 12 below).

- Do manufactured buildings bearing the DBPR Insignia require a plan review and approval by a local building department?

No. Chapter  553.80(1)(d) states, “Building plans approved under s. 553.77(3) and state-approved manufactured buildings, including buildings manufactured and assembled offsite and not intended for habitation, such as lawn storage buildings and storage sheds, are exempt from local code enforcing agency plan reviews except for provisions of the code relating to erection, assembly, or construction at the site. Erection, assembly, and construction at the site are subject to local permitting and inspections. Lawn storage buildings and storage sheds bearing the insignia of approval of the department are not subject to s. 553.842. Such buildings that do not exceed 400 square feet may be delivered and installed without need of a contractor’s or specialty license.

- Are additional inspections of a manufactured (modular) building required by the local building department?

No. Section 553.37(6) F.S. states: Manufactured buildings which have been issued and bear the insignia of approval pursuant to this part upon manufacture or first sale shall not require an additional approval or insignia by a local government in which they are subsequently sold or installed. Buildings or structures that meet the definition of “open construction” are subject to permitting by the local jurisdiction and are not required to bear insignia. However, s. 553.80 (1)(d) FS states. “ Erection, assembly, and construction at the site are subject to local permitting and inspections.”

- Can a manufactured (modular) building be treated any differently than a conventionally constructed (site built) building?

No. Section 553.38 FS, states: Application and scope.—The department shall enforce             every provision of the Florida Building Code adopted pursuant hereto, except that local land use and zoning requirements, fire zones, building setback requirements, side and rear yard requirements, site development requirements, property line requirements, subdivision control, and onsite installation requirements, as well as the review and regulation of architectural and aesthetic requirements, are specifically and entirely reserved to local authorities. Such local requirements and rules which may be enacted by local authorities must be reasonable and uniformly applied and enforced without any distinction as to whether a building is a conventionally constructed or manufactured building. A local government shall require permit fees only for those inspections actually performed by the local government for the installation of a factory-built structure. Such fees shall be equal to the amount charged for similar inspections on conventionally built housing.

- Can a newly manufactured (modular) building be installed anywhere in Florida?

Yes, provided it bears the DBPR insignia and meets the provisions above.

- Can a new factory-built public school building be constructed of Type III-B or V-B (combustible) materials?

No. Section 423.27.3 FBC, states:  “All new portables (relocatables) constructed, purchased or    otherwise acquired by a board shall be noncombustible Type I, II or IV construction.”

- Do the FBC High Velocity Hurricane Zone (HVHZ) requirements apply to a manufactured (modular) building bearing a DBPR insignia?

Yes, if the building was constructed after March 01, 2002, and is intended for sale/installation in Broward or Miami-Dade Counties.  Plans should reflect whether or not approved for HVHZ installation.

- Do windborne debris requirements apply to modular buildings?

Yes, except where specifically exempt, as in storage sheds of 720 square feet or less in size.

- Can a modular residence be installed on a foundation of pads, dry stacked blocks and tie-downs?

This is a site related issue, subject to code interpretation and enforcement by the     local building official as to whether engineered foundation plans comply with the FBC.

- Who can install a modular residence?

Chapter 489 FS requires that a Florida licensed contractor install a modular residence.

- Are Construction Trailers and Temporary Offices required to meet the Florida Building Code

No, 102.2 FBC states “The following buildings, structures and facilities are exempt from the Florida Building Code as provided by law, and any further exemptions shall be as determined by the legislature and provided by law: (d) Temporary buildings or sheds used exclusively for construction purposes. (e) Mobile or modular structures used as temporary offices, except that the provisions of Part II (Section 553.501-553.513, Florida Statutes) relating to accessibility by persons with disabilities shall apply to such mobile or modular structures.

- Do storage sheds have to comply with the FBC?

Yes, storage sheds must comply with the FBC (including local wind design requirements), and bear a DBPR insignia, but are exempt from s.553.842, F.S., State Product Approval System, and windborne debris requirements (720sq.ft or less).

EXISTING MANUFACTURED (MODULAR) BUILDINGS

- How does a local government permit an existing manufactured (modular) building which is being moved to a new jurisdiction, without a copy of the plans and an existing DCA/DBPR insignia?

If the original approved plans are not available, the following options exist:

1) The owner may request a copy of the plans from the manufacturer or Third Party Agency.
2) The owner may obtain a copy of the plans from the Department’s online electronic files, if available. 
3) An “as built” set of plans may be prepared, signed and sealed by a Florida registered engineer or architect, and a Certificate of Occupancy issued by the local building official, in lieu of a DCA/DBPR insignia.

- Does a manufactured building produced under the old Standard Building Code (SBC) have to be upgraded to the new requirements of the Florida Building Code (FBC)?

No, unless it is altered or has a change in occupancy or moved to a location with a higher wind speed requirement

- If an existing manufactured (modular) building is moved to a new location ,what conditions apply?

The relocated building shall comply with wind speed requirement of the new location. If the existing building was manufactured in compliance with the Standard Building Code (SBC), the SBC wind speed map shall be applicable.  If the existing building was manufactured in compliance with the FBC, the FBC wind speed map shall be applicable.

- Can an existing modular building be modified or renovated?

Yes. Onsite modifications to existing modular buildings must be permitted and inspected by the jurisdiction having authority (local building department).  Original plans (or “As-built” plans) and any modifications must be provided to the local building department. When inspected by the local building department, the DCA/DBPR Insignia shall be removed and returned to DBPR, and a Certificate of Occupancy be issued in lieu of the DCA/DBPR Insignia.  Recertification by the local jurisdiction is only applicable for that jurisdiction (not statewide).

- Can an existing modular building be converted from one occupancy type to another?

Yes, the building must be brought into compliance with the current edition of the FBC.   The local jurisdiction has superseding authority over any onsite modifications.

- How do I know if a manufactured (modular) building is DCA/DBPR approved?

All DCA/DBPR approved buildings must bear the DCA/DBPR (State) Insignia affixed to the inside cover of the electric panel or inside the building for those with no electrical panels. Prior to October 1, 2011 the insignia will have the following information:  State of Florida Seal, the Department of Community Affairs, Occupancy, and a MB number, unique to that occupancy type.

October 1, 2011 and after, the insignia will have the following information:  State of Florida Seal, Florida Department of Business and Professional Regulation, Insignia Number, Plan Tracking Number, Manufacturer, Inspection Agency, Occupancy, Number of Modules, and Date of Manufacturer.

The manufacturer shall affix a data plate containing the following information: manufacturer’s name, date of manufacture, serial number, occupancy, construction type, wind velocity, floor load, etc., to the electrical panel or inside the storage shed. 


ADDITIONAL MANUFACTURED (MODULAR) BUILDING INFORMATION

- What is the purpose of the “Certification Letter”?

The Certification Letter attests to the currency (active or inactive status) of the manufacturer only. It does not attest to the currency of the constructions plans or compliance of the building with the latest version of the FBC.  In cases where manufacturers who have closed their businesses; have been purchased and/or changed names; and no copies of the Certification Letter still exist, validation that the manufacturer was once certified can be obtained by performing an “Organizational Search” on the Manufactured (Modular) Buildings website.

- How do I view issued insignias and electronically transmitted plans?

Follow these steps:  Go to: www.floridabuilding.org ,click on the Manufactured (Modular) Buildings icon, click on either “Find an Insignia” or “Find a Plan”, as applicable.