Swimming Pools Technical Advisory Committee – Comment

6th Edition (2017) Florida Building Code, Building

SW – Comment #1

From: Grant Dostie [mailto:gdostie@trentcotney.com] 
Sent: Friday, February 24, 2017 3:35 PM
To: Madani, Mo
Subject: Comment for proposed code modification SW7074-A1

Good afternoon Mo,

Please find attached our comment for proposed code modification SW7074-A1. We have revised our previous comment which you had confirmed receipt.

Please feel free to contact me with any questions.

Thanks You

Grant R. Dostie

Associate Attorney Ph. 813.579.3278 ext. 302 ¦ Fx. 813.902.7612 407 N. HOWARD AVE ¦ SUITE 100 ¦ TAMPA, FL 33606 WWW.TRENTCOTNEY.COM ¦ LIKE US ¦ FOLLOW US

See Attachment #1

TAC Recommendation:  Mr. O’Connor entered a motion for a negative roll call.  Mr. Fardelmann seconded the motion.  The motion passed unanimously with a vote of 9 to 0.

Commission Action:

SW – Comment #2

| # to: | ## Florida Building Commission |
| --- | --- |
| # from: | ## Jennifer Hatfield, representing the Florida Swimming Pool Association |
| # subject: | ## Comments to Public Pool provisions in proposed 6th edition |
| # date: | ## February 24, 2017 |

Item 1: In modification #7058, the TAC deleted the definition of the term ‘Modification’ and in modification #7070 that term was placed with the terms ‘Repairs or Alterations.’ However, upon further review, the term ‘Modification’ was left in two other places of the FBC: sections 454.1.2.3.5 item #7 and 454.1.3.3.6.  Now that this term is no longer applicable, it should be removed and replaced in those two sections as follows:

454.1.2.3.5 Rules and regulations signage. Rules and regulations for bathers shall be installed posted in minimum 1 inch letters which must be legible from the pool deck, and shall contain the following:

1. No food or beverages in pool or on pool wet deck.

2. No glass or animals in the fenced pool area (or 50 feet from unfenced pool).

3. Bathing load: ___ persons.

4. Pool hours: __ a.m. to __ p.m.

5. Shower before entering.

6. Pools of 200 square feet in area or greater without an approved diving well configuration shall have “NO DIVING”, in four inch letters included with the above listed pool rules.

7. Do not swallow the pool water. This statement shall be added to signs at pools that conduct modifications alterations as that term is defined.

454.1.3.3.6 Floating and climb-on devices, objects or toys that are not a part of the approved pool design shall not be tethered in the pool or installed without a engineering modification alterations application and department’s approval.

TAC Recommendation:  Mr. O’Connor entered a motion to approve the comment.  Mr. Pabst seconded the motion.  The motion passed unanimously with a vote of 9 to 0.

Commission Action:

Item 2: In modification #7014, approved by the TAC, at 454.1.6.5.16.6 item #3 requires UV disinfection equipment to be US EPA validated when used in three ‘higher risk facilities’, and one of these ‘higher risk facilities’ is an Interactive Water Feature (aka: a splash pad).  In modification #7074, also approved by the TAC, at 454.1.9.8.6.1, it allows the DIRECT-filtered Interactive Water Feature to use UV disinfection equipment that is certified under the NSF Standard 50, so we have a contradiction between these two code sections.  The revision in 454.1.9.8.6.3 requires US EPA validation for the INDIRECT-filtered IWFs. To assure that the contradiction is correctly applied for design and building IWFs, the following should be added after 454.1.6.5.16.6 item #3 (new proposed language in red):

454.1.6.5.16.6 Ultraviolet (UV) light disinfectant equipment may be used as supplemental water treatment on public pools and additional treatment on interactive water features (IWFs)] subject to the conditions of this paragraph and manufacturer's specifications. UV is encouraged to be used to eliminate or reduce chlorine-resistant pathogens, especially the protozoan cryptosporidium.

1. UV equipment and electrical components and wiring shall comply with the requirements of the National Electrical Code and the manufacturer shall provide a certification of conformance to the jurisdictional building department.

2. UV equipment shall meet UL standards and shall be electrically interlocked with recirculation pump(s) on all pools and with feature pumps(s) on an IWF such that when the UV equipment fails to produce the required dosage as measured by an automated sensor, the feature pump(s) are disabled so the water features do not operate.

3. UV equipment used in higher risk facilities such interactive water features, wading pools, and activity pools shall be validated by a capable party that it delivers the required and predicted UV dose at the validated flow, lamp power and water UV transmittance conditions, and has complied with all professional practices summarized in the USEPA Ultraviolet Disinfectant Guidance Manual dated November 2006, which is publication number EPA 815-R-06-007 available from the department at http://www.floridashealth.org/Environment/ water/swim/index.html or at

http://www.epa.gov/safewater/disinfection/lt2/ pdfs/guideit2_uvguidance.pdf.  Exception: Not applicable when 454.1.9.8.6.1 alternative is used.

4. UV equipment shall constantly produce a validated dosage of at least 40 mJ/cm2 (millijoules per square centimeter) at the end of lamp life.

5. The UV equipment shall not be located in a side stream flow and shall be located to treat all water returning to the pool or water features.

TAC Recommendation:  Mr. O’Connor entered a motion to approve the comment.  Mr. Williams seconded the motion.  The motion passed unanimously with a vote of 9 to 0.

Commission Action:

Item 3: In section 454.1 definitions there is a second ‘wading pool’ definition called ‘wade pool’ that is the exact wording used as and intended for what is listed as the ‘water slides’ definition.  Therefore; this second ‘wade pool’ definition should be deleted.

"Wading pool" means a shallow pool designed to be used by children.

"Water slides" means a water recreation attraction ride which is characterized by having trough-like or tubular flumes or chutes.

"Wade pool" means a water recreation attraction ride which is characterized by having trough-like or tubular flumes or chutes.

TAC Recommendation:  Mr. O’Connor entered a motion to approve the comment.  Mr. Pabst seconded the motion.  The motion passed unanimously with a vote of 9 to 0.

Commission Action:

Item 4: In section 454.1.2.3.3, the metric length for 5 feet and for 18 inches are both listed as 1524 mm.

# 454.1.2.3.3 Lane markings. Pools that are not intended to be utilized for officially sanctioned competition may install lap lane markings provided they meet the following criteria: the markings must be 2 to 6 inches (51 to 152 mm) wide, they must terminate 5 feet (1524 mm) from the end wall in a "T" with the "T" bar at least 18 inches (1524 457.2 mm) long, they must be placed at 7-foot (2134 mm) intervals on center and be no closer than 4 feet (1219 mm) from any side wall, steps or other obstructions. Floating rope lines associated with lap lanes must not obstruct the entrance or exit from the pool and are prohibited when the pool is open for general use.

TAC Recommendation:  Mr. O’Connor entered a motion to approve the comment.  Mr. Pabst seconded the motion.  The motion passed unanimously with a vote of 9 to 0.

Commission Action:

SW – Comment #3

From: Jim [mailto:jel@wetengineering.net] 
Sent: Tuesday, February 28, 2017 3:02 PM
To: Madani, Mo
Subject: Jim LePetrie / WET Engineering Comments

#7073

454.1.2.4:  Para 1, last sentence, the word "line" should be inserted after the word "tile", and the word "tile" should be inserted after the word "each."  The way it's written doesn't make sense.

TAC Recommendation:  Mr. O’Connor entered a motion to approve the comment.  Mr. Williams seconded the motion.  The motion passed unanimously with a vote of 9 to 0.

Commission Action:

#6584

454.1.2.8.2:  This whole section isn't written with our Florida rollout gutters in mind, unfortunately.  The language requires "No-Entry" markers on the deck where the vertical distance between the coping or wet deck and the shelf floor adjacent to the wall is more than 12 inches.  In a rollout gutter pool, assuming the minimum water depth of 8", this vertical distance will be approximately 12" to 14" depending on where it is measured on the gutter surface (since the gutter slopes).  We have varianced 2 of these designs so far, and in my coordination with FDOH we are allowed to use the gutter as a step.  The proposed language's intent is to prohibit too much of a step down; but with the gutter as a step, the distance is broken into two, with both steps at 8" or less.  So we are showing typical depth markers.  I think something should be inserted that allows for this for rollout gutter pools.

Also,  we are not being required to provide "Depth markers of the adjacent pool depth at the sun shelf edge, and no-dive markers shall be placed on the sun shelf floor, every 10 feet or less...above the deeper pool" where the shelf transitions to steps down to the deeper pool.  I think some language should be inserted that exempts the requirement for these markers where steps are used as an access point between the shelf and pool area.

454.1.2.8.3:  Same issue here.  If we can use a gutter as a step, then an additional stair shouldn't be required as the language implies.  I have already coordinated this with FDOH and we are not being required to add a step if the vertical distance taken by a bather entering or leaving the shelf by using the gutter as a step is at or less than the standard 10" pool step riser height.  We are specifying handrails which this language requires.

#7014

454.1.6.5.16.6:  Item 3, first sentence implies ALL UV equipment used for IWF's must be a validated system, which is a direct contradiction with #7074 which allows for only a NSF-50 compliant UV system for the feature flow return.

#7074

Frankly, we are really confused by the proposed language here.

454.1.9.8.6.1:  This makes sense, but the water should not be chemically treated BEFORE the UV unit, but rather AFTER.  This is good practice.

454.1.9.8.6.2:  Where it says "...excess water not required by the features shall be returned to the collector tank..."  does this mean a separate filter pump can be used or does this mean a single feature/filter pump system with a branched return back to the tank?

454.1.9.8.6.3:  Where it says "...before any of this treated water is piped to the water features", does this mean a separate filter pump that returns all flow back to the tank after UV treated with a validated system can be used?  Use of the word "before" implies to me that all of this treatment is in the feature pump return loop if there are separate pumps.

#7070

454.1.10.1:  As we discussed, where it says "...or equivalent replacement of equipment are considered repairs or alterations", what does "equipment" mean?  If this means equivalent replacement of something as mundane as a 1-hp pump motor replacement requires a permit, then that is ridiculous.  I know it's up to the building commission to regulate what gets permitted but a like-for-like replacement really shouldn't need to be permitted.  It's a waste of everyone's time & money.

That's it; please let me know if you wish to discuss any of this.  Thanks for your review.

Thanks,

Jim LePetrie, P.E.

4337 Pablo Oaks Court, Suite 101

Jacksonville, Florida 32224

(904) 223-9773

(866) 832-9236 fax

jel@wetengineering.net

www.wetengineering.net

See Attachment # 2

TAC Recommendation:  Mr. Vincent entered a motion to accept the language on balance of comment #3 (as noted in Attachment #2) excluding the first section.  Mr. O’Connor seconded the motion.  The motion passed unanimously with a vote of 9 to 0.

Commission Action:

SW – Comment #4

From: Vincent, Bob G [mailto:Bob.Vincent@flhealth.gov] 
Sent: Tuesday, February 14, 2017 11:48 AM
To: Sellers, Chip; Madani, Mo; Campbell, Thomas; Hammonds, April; Duval, Nick
Subject: RE: Swimming Pool TAC Avalability February 28th 10;00 A.M.

Sorry I hadn’t sent anything before today.   I have no good excuse, just uber busy.  I noticed two minor glitches in the 2015 FBC, plus two more important ones in the 6th ed. 2017 revision:

In the 2015 FBC section 454.1 definitions there is a second ‘wading pool’ definition called ‘wade pool’ that is actually the ‘water slides’ definition.  This second ‘wade pool’ definition should be deleted.

In section 454.1.2.3.3, the metric length for 5 feet and for 18 inches are both listed as 1524 mm.

For the new 2017 6th edition FBC 454.1, here is a question if this first one is a glitch, and maybe the second one is a glitch too:

Under revision #7058, we deleted the definition of the term ‘Modification’ to add the terms ‘Repairs or Alterations’ at 454.1.10.1 under revision #7070, but we left ‘Modification’ in two other places of the FBC:  at 454.1.2.3.5 item # 7. Which was intended in DOH rule to add this pool rules sign statement whenever a modification is dine.   ‘Modification’ it is also still found at 454.1.3.3.6

The revision #7014 at 454.1.6.5.16.6 item #3 requires UV disinfection equipment to be US EPA validated when used in three ‘higher risk facilities’, and one of these ‘higher risk facilities’ is an Interactive Water Feature (aka: a splash pad).   In revision #7074, at 454.1.9.8.6.1 , we allowed the DIRECT-filtered Interactive Water Feature to use UV disinfection equipment that is certified under the NSF Standard 50, so we have a contradiction between these two code sections.    The revision in 454.1.9.8.6.3 requires US EPA validation for the INDIRECT-filtered IWFs.

To assure that the contradiction is correctly applied for design and building IWFs, you could add a NOTE or EXCEPTION after 454.1.6.5.16.6 item #3 that states: Not applicable when 454.1.9.8.6.1 alternative is used.  If this not possible, a DEC Statement could/should be issued.

Bob Vincent, Environmental Administrator, FL Dept of Health, Div. of Disease Control and Health Protection, Bur. of Environmental Health, Water Programs, Tallahassee, FL  office 850.245.4578, cell 850.251.7510  Webpage: http://www.floridahealth.gov/healthy-environments/index.html

TAC Recommendation:  Withdrawn by Mr. Vincent due to the fact that his concerns were covered under Comment 2 and 3.

Commission Action: