Submitted by Steve Strawn

Comment 1: “Labeled” should be italicized because this needs to be consistent with referenced definitions. By code labeled has a very clear meaning.

Section R612.3 Testing and labeling. Revise to read as shown:

R612.3 Testing and labeling. Exterior windows and doors shall be tested by an approved independent testing laboratory, and shall be labeled to indicate compliance with AAMA/WDMA/CSA 101/I.S.2/A440 or TAS 202 (HVHZ shall comply with TAS 202).  Exterior side-hinged doors shall be tested and labeled as conforming to AAMA/WDMA/CSA 101/I.S.2/A440 or comply with Section R612.5.Exterior windows and doors shall be labeled with a permanent label, marking, or etching providing traceability to the manufacturer and product. The following shall also be required either on a permanent label or on a temporary supplemental label applied by the manufacturer: information identifying the manufacturer, the product model/series number, positive and negative design pressure rating, product maximum size, glazing thickness, impact-resistance rating if applicable, Florida Product Approval number or Miami-Dade Product Approval number, applicable test standard(s), and approved product certification agency, testing laboratory, evaluation entity or Miami-Dade Product Approval. 

The labels are limited to one design pressure rating per reference standard. The temporary supplemental label shall remain on the window or door until final approval by the building official.

Exceptions:

No change

Comment #2: Added tested to be consistent with each of the other bullet points

R612.3.1 Comparative analysis. Structural wind load design pressures for window and door units other than the size tested in accordance with Section R612.3 shall be permitted to be higher different than the design value of the tested unit provided such different pressures are determined by accepted engineering analysis or validated by an additional test of the window or door unit to the different design pressure in accordance with section R612.3. All components of the alternative size unit shall be the same as the tested or labeled unit.

Exceptions:

i Operable windows and doors rated in this manner shall comply with the following:

1. The frame area of the alternate size unit shall not exceed the frame area of the tested approved unit.

2. Shall vary from the tested approved unit only in width, height or load requirements.

3. Shall not exceed 100 percent of the proportional deflection for fiber stress of the intermediate members of the tested approved unit.

4. Shall not exceed 100 percent of the concentrated load at the juncture of the intermediate members and the frame of the tested approved unit.

5. Shall not exceed the air and water infiltration resistance of the tested approved unit.

6. Shall not exceed the maximum cyclic pressure of the tested approved unit when tested per TAS 201 and TAS 203 or ASTM E 1886 and ASTM E 1996 where applicable.

ii. Non-operable windows and doors rated in this manner shall comply with the following:

1. The frame area of the alternate size unit shall not exceed the frame area of the tested approved unit.

2. Shall vary from the tested approved unit only in width, height or load requirements.

3. The maximum uniform load distribution (ULD) of any side shall be equal to the uniform load carried by the side divided by the length of the side.

4. The ULD of any member shall not exceed the ULD of the corresponding member of the tested approved unit.

5. The ULD of each member shall be calculated in accordance with standard engineering analysis.

6. Shall not exceed the air and water infiltration resistance of the tested approved unit.

7. Shall not exceed the maximum cyclic pressure of the tested approved unit when tested per TAS 201 and TAS 203 or ASTM E 1886 and ASTM E 1996 where applicable.

Comment #3:   Section R612.6.1 should be held in reserve but delete the language. The (proposed) charging Section R612.6 covers Wind -borne debris protection and references Section R301.2.1.2 Protection of openings and the specific test requirements related to windborne debris protection. Section R612.3 Testing and labeling includes requirements to provide labeling for impact resistance ratings if applicable. It is my belief that retaining the language as provided in Section R612.6.1 would cause an additional label to be provided showing the impact test standards and the applicable rating. This is already covered by R612.3. Manufacturers should not have to provide an additional label but should provide the information when applicable as suggested in R612.3 as written.

I am in complete agreement with staff recommendation on Tracking ID S2 and as noted below.

R612.6 Wind-borne debris protection. Protection of exterior windows and glass doors in buildings located in windborne debris regions shall be in accordance with Section R301.2.1.2.

R612.6.1 Fenestration testing and labeling. Fenestration shall be tested by an approved independent laboratory, listed by an approved entity, and bear a label identifying manufacturer, performance characteristics, and approved inspection agency to indicate compliance with the requirements of the following specification:

1. ASTM E 1886 and ASTM E 1996; or

2. AAMA 506.  Reserved