508.3.1 Accessory occupancies. Accessory occupancies are those occupancies subsidiary to the main occupancy of the building or portion thereof. Aggregate accessory occupancies shall not occupy more than 10 percent of the area of the story in which they are located and shall not exceed the tabular values in Table 503, without height and area increases in accordance with Sections 504 and 506 for such accessory occupancies.

Exceptions:

1.	Accessory assembly areas having a floor area less than 750 square feet (69.7 m2) are not considered separate occupancies.

2.	Assembly areas that are accessory to Group E occupancies are not considered separate occupancies except when applying the assembly occupancy requirements of Chapter 11.

3.	Accessory religious educational rooms and religious auditoriums with occupant loads of less than 100 are not considered separate occupancies.