R613.3 Exterior windows, sliding and patio glass doors.

R613.3.1 Testing and labeling. Exterior windows and glass doors shall be tested by an approved independent testing laboratory, and shall be labeled with an approved permanent label identifying the manufacturer, the products model/series number, performance characteristics and approved product certification agency, testing laboratory, evaluation entity or Miami-Dade Product Approval to indicate compliance with the requirements of one of the following specifications:

ANSI/AAMA/NWWDA101/I.S.2 or 101/I.S.2/NAFS or AAMA/WDMA/CSA 101/I.S.2/A440 or TAS 202 (HVHZ shall comply with TAS 202 utilizing ASTM E 1300-98 or ASTM E 1300-02).

Exceptions:

1.	Door assemblies installed in nonhabitable areas where the door assembly and area are designed to accept water infiltration need not be tested for water infiltration.

2.	Door assemblies installed where the overhang (OH) ratio is equal to or more than 1 need not be tested for water infiltration. The overhang ratio shall be calculated by the following equation:

OH ratio	 = 	OH length/OH height

Where:

OH length 	= 	The horizontal measure of how far an overhang over a door projects out from door's surface.

OH height 	= 	The vertical measure of the distance from the door's sill to the bottom of the overhang over a door.

3.	Pass-through windows for serving from a single-family kitchen, where protected by a roof overhang of 5 feet (1.5 m) or more shall be exempted from the requirements of the water infiltration test.

4.	Decorative glazed openings.

The permanent label is limited to only one design pressure rating per reference standard per label. Exterior windows and glass doors shall be labeled with a temporary supplemental label printed and applied by the manufacturer. The label shall identify the manufacturer, products model/series number, positive and negative design pressure rating, products maximum size, glazing thickness, indicate impact rated if applicable, Florida Product Approval or Miami-Dade Product Approval number if applicable, and applicable test standard. The supplemental label is limited to only one design pressure rating per reference standard per label. This supplemental label shall remain on the window until final approval by the building official.

The permanent label is always the default label in case the temporary label is missing or no longer legible for final approval by the building official.

Glass Strength: Products tested and labeled as conforming to ANSI/AAMA/NWWDA 101/I.S.2 or 101/I.S.2/NAFS or AAMA/WDMA/CSA 101/I.S.2/A440 or TAS 202 shall not be subject to the requirements of the Florida Building Code, Building. Determination of load resistance of glass for specific loads of products not tested and certified in accordance with Section R613.3.1 shall be designed and labeled to comply with ASTM E 1300. The supplemental label shall designate the type and thickness of glass or glazing material.

R613.3.1.1 Testing and labeling of skylights. Exterior skylights shall be tested by an approved independent testing laboratory, and shall be labeled with an approved permanent label identifying the manufacturer, the products model/series number, performance characteristics and approved product evaluation entity to indicate compliance with the requirements of the following specification:

AAMA/WDMA 101/I.S.2/NAFS, Voluntary Performance Specification for Windows, Skylights and Glass Doors, or TAS 202 (HVHZ shall comply with TAS 202).

The permanent label is limited to only one design pressure rating per reference standard per label.

Exterior skylights shall be labeled with a temporary supplemental label printed and applied by the manufacturer. The label shall identify the manufacturer, products model/series number, positive and negative design pressure rating, products maximum size, glazing thickness, indicate impact rated if applicable, Florida Product Approval or Miami-Dade Product Approval number if applicable, and applicable test standard. The supplemental label is limited to only one design pressure rating per reference standard per label. This supplemental label shall remain on the window until final approval by the building official. The permanent label is always the default label in case the temporary label is missing or no longer legible for final approval by the building official.

R613.3.2 Supplemental label. A temporary supplemental label conforming to AAMA 203, Procedural Guide for the Window Inspection and Notification System, shall be acceptable for establishing calculated allowable design pressures higher than indicated on the label required by Section R613.3.1 for window sizes smaller than that required by the ANSI/AAMA/NWWDA 101/I.S.2 or 101/I.S.2/NAFS or AAMA/WDMA/CSA 101/I.S.2/A440 test requirements. This supplemental label shall remain on the window until final approval by the building official.

Exception 1:

Comparative analysis of operative windows and glazed doors may be made, provided the proposed unit complies with the following:

1.	Shall always be compared with a tested and currently approved unit.

2.	Varies only in width, height and/or load requirements.

3.	Shall not exceed 100 percent of the proportional deflection for fiber stress of the intermediate members of the approved unit.

4.	Shall conform as to extruded members, reinforcement and in all other ways with the tested approved unit.

5.	Shall not exceed 100 percent of the concentrated load at the juncture of the intermediate members and the frame of the approved unit.

6.	Shall not permit more air and water infiltration than the approved unit based on the height above grade.

7.	Compared unit shall not exceed the maximum cyclic pressure when tested per TAS 203 or ASTM E 1886 and ASTM E 1996.

Exception 2:

Comparative analysis of fixed glass windows may be made, provided the proposed unit complies with the following:

1.	Shall always be compared with a tested and currently approved unit.

2.	Varies only in width, height and/or load requirements.

3.	The design is identical in all respects. e.g., extrusions, glazing system, joinery, fasteners, etc.

4.	Shall not permit more air and water infiltration than the approved unit based on height above grade.

5.	The maximum uniform load distribution (ULD) of any side is equal to the uniform load carried by the side divided by the length of the side.

6.	The ULD of any member must not exceed the ULD of the corresponding member of the tested window.

7.	The uniform load distribution on each member shall be calculated in accordance to Section 2, Engineering Design Rules, of the AAMA 103.3 Procedural Guide.

8.	Compared unit shall not exceed the maximum cyclic pressure when tested per TAS 203 or ASTM E 1886 and ASTM E 1996.