2010 Florida Building Code, Existing Building

Glitch Supplement

Chapter 2 - DEFINITIONS

Revise the following definition to read as follows:

SUNROOM

1. A room with roof panels that include sloped glazing that is a one-story structure added to an existing dwelling with an open or glazed area in excess of 40 percent of the gross area of the sunroom structure’s exterior walls and roof.

2. A one story structure added to a dwelling with structural roof panels without sloped glazing. The sunroom walls may have any configuration, provided the open area of the longer wall and one additional wall is equal to at least 65 percent of the area below 6 foot 8 inches (2032 mm) of each wall, measured from the floor.

2. A one-story structure added to a dwelling with solid roof panels without sloped glazing. The sunroom walls may have any configuration, provided the open areas with operable or fixed glass or windows or side hinged or sliding glass doors of the longer wall and one additional wall is equal to at least 65 percent of the area below 6 foot 8 inches of each wall, measured from the floor.

S4609

Chapter 5 - REPAIRS

[B] SECTION 310.  Revise to read as follows:

ACCESSIBILITY FOR EXISTING BUILDINGS

See the provisions Chapter 11 of the Florida Building Code, Building. Accessibility.

505.1 General. Change to read as shown.

505.1 General. Repairs shall be done in accordance with the provisions Chapter 11 of the Florida Building Code, Building.Accessibility.

CA4733

Section 605, Accessibility

605.1 General. Change to read as shown.

605.1 General. 1 Accessibility shall be in accordance with the provisions of the Florida Building Code, Accessibility with Chapter 11 of the Florida Building Code, Building.

CA4734

611.3, Exception 5 1507.15.3 Roof Coating.  Add to read as follows:

5. 1507.15.3 Roof Coating.  Application of elastomeric and or maintenance coating systems over existing asphalt shingles shall be in accordance with the shingle manufacturer’s approved installation instructions. {3814}

Comment after Glitch

611.7.1 Roof decking attachment for site-built single-family residential structures.  Revise to read as follows:

611.7.1 Roof decking attachment for site-built single-family residential structures. For site-built single-family residential structures the fastening shall be in accordance with Section 611.7.1.1 or 611.7.1.2 as appropriate for the existing construction. 8d nails shall be a minimum of 0.131 0.113 inch (3.3 mm 2.9 mm) in diameter and shall be a minimum of 21/4-inch (57 mm) long to qualify for the provisions of this section for existing nails regardless of head shape or head diameter.

R4811

TABLE 611.7.1.2 – Change to read as shown.

TABLE 611.7.1.2

Change table headings as follows:

Vasd Wind speed 110 mph or less supplemental fastener spacing shall be no greater than

Vasd Wind speed greater than 110 mph supplemental fastener spacing shall be no greater than

Add new Note c to read as follows:

c. Vasd shall be determined in accordance with Section 1609.3.1 of the Florida Building Code, Building or Section R301.2.1.3 of the Florida Building Code, Residential.

S4610

TABLE 611.8.1 – Change to read as shown.

TABLE 611.8.1

In table heading change “Basic Wind Speed” to “Ultimate Design Wind Speed, Vult.”

Add new Note c and d as follows:

c. For Ultimate design wind speeds, Vult greater than 170 mph, wind uplift forces shall be determined in accordance with Florida Building Code, Residential, Section R802.3 or ASCE 7.

d. Ultimate Design Wind Speeds determined from Figure 1609A in the Florida Building Code, Building or Figure R301.2(4) in the Florida Building Code, Residential.

R4611

Chapter 7 – ALTERATIONS – LEVEL 2

Section 706, Accessibility

706.1 General.    Revise text to reads as shown.

706.1 General. A building, facility, or element that is altered shall comply with the provisions of the Florida Building Code, Accessibility Chapter 11 of the Florida Building Code, Building.

F4735

Chapter 8 – ALTERATIONS – LEVEL 3

Section 806, Accessibility

806.1 General. Change to read as shown.

806.1 General. A building, facility, or element that is altered shall comply with and provisions of the Florida Building Code, Accessibility Chapter 11 of the Florida Building Code, Building.

CA4736

Chapter 9 – CHANGE OF OCCUPANCY

906.1 – Change to read as shown.

Section 906, Accessibility

906.1 General. Accessibility in portions of buildings undergoing a change of occupancy classification shall comply with and provisions of the Florida Building Code, Accessibility Chapter 11 of the Florida Building Code, Building.

F4737

912.8 Accessibility. Change to read as shown.

912.8 Accessibility. Existing buildings or portions thereof that undergo a change of group or occupancy classification shall comply with the  provisions of the Florida Building Code, Accessibility with Chapter 11 of the Florida Building Code, Building.

F4738

Chapter 10 - ADDITIONS

1005.1 Minimum requirements. Change to read as shown. 1005.1 Minimum requirements. Accessibility provisions for new construction shall apply to additions. An addition that affects the accessibility to, or contains an area of, primary function shall comply with the requirements of the Florida Building Code, Accessibility in Chapter 11 of the Florida Building Code, Building.

F4739

Chapter 11 – HISTORIC BUILDINGS

SECTION 1102 – Change the following definition to read as shown:

HISTORIC BUILDING. For the purposes of this code and the referenced documents, an historic building is defined as a building or structure that is:

1.            Individually listed in the National Register of Historic Places; or

2.            A contributing property in a National Register of Historic Places listed district; or

3.            Designated as historic property under an official municipal, county, special district or state designation, law, ordinance or resolution either individually or as a contributing property in a district; or

4.            Determined eligible by the Florida State Historic Preservation Officer for listing in the National Register of Historic Places, either individually or as a contributing property in a district.

For accessibility requirements, see the Florida Building Code, Accessibility Building , Chapter 11, Section 11-4.1.7, Accessible buildings: historic preservation .

CA4740

Chapter 15 – REFERENCED STANDARDS

ASME – Change the following standard to read as shown.

ASME

American Society of Mechanical Engineers

Three Park Avenue

New York, NY 10016-5990

Standard reference number               Title                                                                                        Referenced in code section number

REVISE text as shown.

A17.3—96                                    Safety Code for Existing Elevators and Escalators                                 802.1.2

A17.1/CSA B44-2007                        Safety Code for Elevators and Escalators includes A17.1a- 2008  and A17.1b-2009 Addenda,               310.8.2, 605.1.2, 802.1.2

A18.1—08                       Safety Standard for Platform Lifts and Stairway Chairlifts       310.8.3, 605.1.3

SP4730

Chapter 16 - RETROFITTING

TABLE 1604.2 – Change to read as shown

TABLE 1604.2

In table heading change:

Maximum Vasd 3-Sec Gust Basic Wind Speed a,d

Add new Note d to read as follows:

d. Vasd shall be determined in accordance with Section 1609.3.1 of the Florida Building Code, Building or Section R301.2.1.3 of the Florida Building Code, Residential

R4612

TABLE 1604.5.1 – Change to read as shown.

TABLE 1604.5.1

In table heading change:

Maximum Vasd 3-Sec Gust Basic Wind Speed a

Add new Note a to read as follows:

- Vasd shall be determined in accordance with Section 1609.3.1 of the Florida Building Code, Building or Section R301.2.1.3 of the Florida Building Code, Residential.

R4613

TABLE 1604.5.2 – Change to read as shown.

TABLE 1604.5.2

In table heading change:

Maximum Vasd 3-Sec Gust Basic Wind Speed a

Add new Note a to read as follows:

- Vasd shall be determined in accordance with Section 1609.3.1 of the Florida Building Code, Building or Section R301.2.1.3 of the Florida Building Code, Residential.

R4614

TABLE 1605.2 – Change to read as shown.

TABLE 1605.2

In table heading change

Ultimate Design Basic Wind Speed, Vult.

Add new Notes a and b as follows:

a. For Ultimate design wind speeds, Vult greater than 150 mph, uplift connection loads shall be determined in accordance with ASCE 7.

b. Ultimate Design Wind Speeds determined from Figure 1609A in the Florida Building Code, Building or Figure R301.2(4) in the Florida Building Code, Residential.

R4615