2010 Florida Building Code, Fuel Gas

Glitch Supplement

Chapter 2 – DEFINITIONS

REGULATOR – Change to read as shown.

REGULATOR. A device for controlling and maintaining a uniform gas supply pressure, either pounds to pounds, pounds-to-inches water column (MP regulator) or inches-to-inches water column (appliance regulator).

P4835

RISER, GAS  – Change to read as shown.

RISER, GAS.  A vertical pipe supplying fuel gas to a meter assembly or a pressure regulator.

P4834

UTILITY GASES – Change to read as shown.

UTILITY GASES. Natural gas, manufactured gas, liquefied petroleum gas-air mixture or mixtures of any of these gases.

P4836

Chapter 3 – GENERAL REGULATIONS

301.11 Flood hazard – Change to read as shown.

301.11 Flood hazard. For structures located in flood hazard areas, the appliance, equipment and system installations regulated by this code shall be located at or above the elevation required by Section 1612.4 of the Florida Building Code, Building for utilities and attendant equipment design flood elevation and shall comply with the flood-resistant construction requirements of the Florida Building Code.

Exception: The appliance, equipment and system installations regulated by this code are permitted to be located below the elevation required by Section 1612.4 of the Florida Building Code, Building for utilities and attendant equipment provided that they are designed and installed to prevent water from entering or accumulating within the components and to resist hydrostatic and hydrodynamic loads and stresses, including the effects of buoyancy, during the occurrence of flooding to such elevation.

P4644, P4754