Comments on the 2013 Update to the Florida Building Code to be considered by the Commission

Code Administration TAC

Meeting Date: August 22, 2013

| Tracking ID | Florida Building Code, Building Proposed Changes | Comments | TAC Action August 1, 2013 | Commission Action |
| --- | --- | --- | --- | --- |
| AC1 | Proponent: Staff [A] 105.2 Work exempt from permit. Exemptions from permit requirements of this code shall not be deemed to grant authorization for any work to be done in any manner in violation of the provisions of this code or any other laws or ordinances of this jurisdiction. Permits shall not be required for the following: | Glitch – (e) Changes to or inconsistencies with federal or state law. The Commission has no authority to address local laws or ordinances. | Glitch – Yes (Unanimous) Technical correction – As Submitted (Unanimous) | Approved as recommended by TAC |
| AC2 | Proponent: Staff [A] 105.2.3 Public service agencies. A permit shall not be required for the installation, alteration or repair of generation, transmission, distribution or metering or other related equipment that is under the ownership and control of public service agencies by established right. Reserved | Glitch – (e) Changes to or inconsistencies with federal or state law. (553.73(10), F.S. | Glitch – Yes (Unanimous) Technical correction – As Submitted (Unanimous) | Approved as recommended by TAC |
| AC3 | Proponent: Staff Change Section 105.5 to read as follows: Option #1 – Reinstate section 105.5 105.5 Expiration. Every permit issued shall become invalid unless the work on the site authorized by such permit is commenced within 180 days after its issuance, or if the work authorized on the site by such permit is suspended or abandoned for a period of 180 days after the time the work is commenced. The building official is authorized to grant, in writing, one or more extensions of time, for periods not more than 180 days each. The extension shall be requested in writing and justifiable cause demonstrated. Reserved. Option #2 – Provisions intended to replace Section 105.5 above. 105.4 Conditions of the permit. 105.4.1 Permit intent. A permit issued shall be construed to be a license to proceed with the work and not as authority to violate, cancel, alter or set aside any of the provisions of the technical codes, nor shall issuance of a permit prevent the building official from thereafter requiring a correction of errors in plans, construction or violations of this code. Every permit issued shall become invalid unless the work authorized by such permit is commenced within six months after its issuance, or if the work authorized by such permit is suspended or abandoned for a period of six months after the time the work is commenced. 105.4.1.1 If work has commenced and the permit is revoked, becomes null and void, or expires because of lack of progress or abandonment, a new permit covering the proposed construction shall be obtained before proceeding with the work. 105.4.1.2 If a new permit is not obtained within 180 days from the date the initial permit became null and void, the building official is authorized to require that any work which has been commenced or completed be removed from the building site. Alternately, a new permit may be issued on application, providing the work in place and required to complete the structure meets all applicable regulations in effect at the time the initial permit became null and void and any regulations which may have become effective between the date of expiration and the date of issuance of the new permit. 105.4.1.3 Work shall be considered to be in active progress when the permit has received an approved inspection within 180 days. This provision shall not be applicable in case of civil commotion or strike or when the building work is halted due directly to judicial injunction, order or similar process. 105.4.1.4 The fee for renewal reissuance and extension of a permit shall be set forth by the administrative authority. 105.5 Expiration. Reserved. | Re-instate. Glitch – (c) Unintended results from the integration of previously adopted Florida-specific amendments with the model code. Recommend Option #2. | Glitch – Yes (Unanimous) Technical correction – Option “2” recommended As Submitted (Unanimous) | Approved as recommended by TAC |
| AC4 | Proponent: Staff [A] 107.3.4 Design professional in responsible charge. When it is required that documents be prepared by a registered design professional, the building official shall be authorized to require the owner to engage and designate on the building permit application a registered design professional who shall act as the registered design professional in responsible charge. If the circumstances require, the owner shall designate a substitute registered design professional in responsible charge who shall perform the duties required of the original registered design professional in responsible charge. The building official shall be notified in writing by the owner if the registered design professional in responsible charge is changed or is unable to continue to perform the duties. The registered design professional in responsible charge shall be responsible for reviewing and coordinating submittal documents prepared by others, including phased and deferred submittal items, for compatibility with the design of the building. Reserved | Glitch – (e) Changes to or inconsistencies with federal or state law. (see 471 and 553.73(2), F.S) | Glitch – Yes (Unanimous) Technical correction – As Submitted (Unanimous) | Approved as recommended by TAC |
| AC5 | Proponent: Staff [A] 107.2.5 Site plan. The construction documents submitted with the application for permit shall be accompanied by a site plan showing to scale the size and location of new construction and existing structures on the site, distances from lot lines, the established street grades and the proposed finished grades and, as applicable, flood hazard areas, floodways, and design flood elevations; and it shall be drawn in accordance with an accurate boundary line survey. In the case of demolition, the site plan shall show construction to be demolished and the location and size of existing structures and construction that are to remain on the site or plot. The building official is authorized to waive or modify the requirement for a site plan when the application for permit is for alteration or repair or when otherwise warranted. [A] 107.2.5.1 Design flood elevations. Where design flood elevations are not specified, they shall be established in accordance with Section 1612.3.1. Add Section 107.2.5.2 as per HB 269 107.2.5.2 For the purpose of inspection and record retention, site plans for a building may be maintained in the form of an electronic copy at the worksite. These plans must be open to inspection by the building official or a duly authorized representative, as required by the Florida Building Code. | Glitch – (e) Changes to or inconsistencies with federal or state law. Section 553.73, F.S. | Glitch - No (Unanimous) No Further Action taken | Glitch - Yes Keep Section 107.2.5 and 107.2.5.1 language and add Section 107.2.5.2 as indicated to comply with HB 269 |
| AC6 | Proponent: Staff [A] 107.3.4.1 Deferred submittals. For the purposes of this section, deferred submittals are defined as those portions of the design that are not submitted at the time of the application and that are to be submitted to the building official within a specified period. Deferral of any submittal items shall have the prior approval of the building official. The registered design professional in responsible charge shall list the deferred submittals on the construction documents for review by the building official. Documents for deferred submittal items shall be submitted to the registered design professional in responsible charge who shall review them and forward them to the building official with a notation indicating that the deferred submittal documents have been reviewed and found to be in general conformance to the design of the building. The deferred submittal items shall not be installed until the deferred submittal documents have been approved by the building official. Reserved | Glitch – (e) Changes to or inconsistencies with federal or state law. (see 471 and 553.73(2), F.S) | Glitch - No (Unanimous) No Further Action taken | Not a Glitch as recommended by TAC |
| AC7 | Proponent: Staff [A] 110.4 Inspection agencies. The building official is authorized to accept reports of approved inspection agencies, provided such agencies satisfy the requirements as to qualifications and reliability. Reserved | Glitch – (e) Changes to or inconsistencies with federal or state law. (see 471 and 553.73(2), F.S) | Glitch - No (Unanimous) No Further Action taken | Not a Glitch as recommended by TAC |
| AC8 | Proponent: Staff Section 553.793, Florida Statutes, is created to read: 553.793 Streamlined low-voltage alarm system installation permitting.— Add Section 105.17 to read as follows (passed by the 2013 Legislature/HB 269): 105.17 Streamlined low-voltage alarm system installation permitting.— (1) As used in this section, the term: (a) "Contractor" means a person who is qualified to engage in the business of electrical or alarm system contracting pursuant to a certificate or registration issued by the department under part II of chapter 489, Florida Statutes. (b) "Low-voltage alarm system project" means a project related to the installation, maintenance, inspection, replacement, or service of a new or existing alarm system, as defined in s. 489.505, Florida Statutes, operating at low voltage, as defined in the National Electrical Code Standard 70, Current Edition, and ancillary components or equipment attached to such a system, including, but not limited to, home-automation equipment, thermostats, and video cameras. (2) Notwithstanding any provision of law this Code, this section applies to low-voltage alarm system projects for which a permit is required by a local enforcement agency. (3) This section does not apply to the installation or replacement of a fire alarm if a plan review is required. (4) A local enforcement agency shall make uniform basic permit labels available for purchase by a contractor to be used for the installation or replacement of a new or existing alarm system at a cost as indicated in s. 553.793, Florida Statutes. of not more than $55 per label per project per unit. However, a local enforcement agency charging more than $55, but less than $175, for such a permit as of January 1, 2013, may continue to charge the same amount for a uniform basic permit label until January 1, 2015. A local enforcement agency charging more than $175 for such a permit as of January 1, 2013, may charge a maximum of $175 for a uniform basic permit label until January 1, 2015. (a) A local enforcement agency may not require a contractor, as a condition of purchasing a label, to submit information other than identification information of the licensee and proof of registration or certification as a contractor. (b) A label is valid for 1 year after the date of purchase and may only be used within the jurisdiction of the local enforcement agency that issued the label. A contractor may purchase labels in bulk for one or more unspecified current or future projects. (5) A contractor shall post an unused uniform basic permit label in a conspicuous place on the premises of the low-voltage alarm system project site before commencing work on the project. (6) A contractor is not required to notify the local enforcement agency before commencing work on a low-voltage alarm system project. However, a contractor must submit a Uniform Notice of a Low-Voltage Alarm System Project as provided under subsection (7) to the local enforcement agency within 14 days after completing the project. A local enforcement agency may take disciplinary action against a contractor who fails to timely submit a Uniform Notice of a Low-Voltage Alarm System Project. (7) The Uniform Notice of a Low-Voltage Alarm System Project may be submitted electronically or by facsimile if all submissions are signed by the owner, tenant, contractor, or authorized representative of such persons. The Uniform Notice of a Low-Voltage Alarm System Project must contain the following information shall be in the format prescribed by the local enforcement agency and must comply with the requirements of s. 553.793(7), Florida Statutes.: UNIFORM NOTICE OF A LOW-VOLTAGE ALARM SYSTEM PROJECT Owner's or Customer's Name................ Owner's or Customer's Address................ City.................... State............ Zip............ Phone Number................ E-mail Address................ Contractor's Name.................... Contractor's Address................................ City.................... State............ Zip............ Phone Number................ Contractor's License Number.................... Date Project Completed........................ Scope of Work................................................ Notice is hereby given that a low-voltage alarm system project has been completed at the address specified above. I certify that all of the foregoing information is true and accurate. ...(Signature of Owner, Tenant, Contractor, or Authorized Representative)... (8) A low-voltage alarm system project may be inspected by the local enforcement agency to ensure compliance with applicable codes and standards. If a low-voltage alarm system project fails an inspection, the contractor must take corrective action as necessary to pass inspection. (9) A municipality, county, district, or other entity of local government may not adopt or maintain in effect an ordinance or rule regarding a low-voltage alarm system project that is inconsistent with this section. (10) A uniform basic permit label shall not be required for the subsequent maintenance, inspection, or service of an alarm system that was permitted in accordance with this section. The provisions of this act are not intended to impose new or additional licensure requirements on persons licensed in accordance with the applicable provisions of chapter 489, Florida Statutes. | Glitch – (e) Changes to or inconsistencies with federal or state law. HB 269. | Glitch – Yes (Unanimous) Technical correction – As Submitted (Unanimous) | Approved as recommended by TAC |