2010 FBC Supplement to the

2007 Test Protocols for High Velocity Hurricane Zones

ROOFING APPLICATION STANDARD (RAS) No. 115-11

STANDARD PROCEDURES FOR ASPHALTIC SHINGLE INSTALLATION

Revise Section 5.4 to read as follows:

Section 5.4

Valley metal shall be a minimum 16 in. wide rolled or preformed material of thickness' in compliance with Section 1517.6 of the Florida Building Code, Building. Valley metal shall be set over the underlayment, or over an optional 18 in. sweat sheet. Fasten valley metal with minimum 12 gage by 11/4 in. annular ring shank roofing nails of similar materials 12 in. o.c. 1 in. in from each exterior edge, except where architectural appearance is to be preserved, in which case a min ¾ in. nail may be used. The entire edge of the flange shall be sealed, covering all nail penetrations with flashing cement and membrane. All horizontal laps shall be a minimum of 6 in. and shall be fully embedded with approved flashing cement. No nails shall be permitted in the center of the valley.    {4362}

Revise Section 7.3 to read as follows:

Section 7.3

Do not nail into or above factory-applied adhesives. Shingle nails shall be located in accordance with the manufacturer’s product approval. Ensure no cutout or end joint is less than 2 inches from a nail in an underlying course. Start nailing from the end nearest the shingle just laid and proceed across. Do not attempt to realign a shingle by shifting the free end after two nails are in place. Drive nails straight so that the edge of the nail head does not cut into the shingle. Nail heads should be driven flush with the shingle surface. Fasteners shall not be improperly driven (see Detail A). {4362}

Revise Section 10.2 to read as follows:

Section 10.2

For ventilator and exhaust stacks located at the ridge, follow the same procedure, but bring the shingles up to the pipe from both sides and bend the flange over the ridge to lie in both roof planes, overlapping the roof shingles at all points. Ridge shingles are then positioned to cover the flange. Embed the ridge shingles in approved flashing cement where they overlap the flange. Roof ventilators and ridge vents shall be installed and flashed in accordance with ventilators their Product Approval. {4362}

Revise Section 11.2 to read as follows:

Section 11.2

Install flashing in accordance with Section 9 above. Apply the base flashing to the low side of the chimney first. Bend the base flashing so that the lower section extends at a minimum of 4 inches over the shingles and the upper section extends a minimum of 12 inches up the vertical face of the chimney. Work the flashing firmly and smoothly into the joint between the shingles and chimney. Set both the roof and chimney overlaps in approved flashing cement placed over the shingles and on the chimney face. The flashing shall be secured in accordance with RAS 111. Use metal step flashing for the sides of the chimney, positioning the units in the same manner as flashing on a vertical side wall, in accordance with Details A and B herein. Secure each flashing unit to the masonry with approved flashing cement and to the deck with approved nails. Embed the end shingles in each course that overlap the flashing in an 8" bed of approved flashing cement. Place the rear base flashing over the cricket and the high side of the chimney. Chimney crickets shall be waterproofed in compliance with the shingle manufacturers published literature. All chimney flashings shall be countered counter flashed. Metal counter flashing shall be in accordance with RAS 111. {4362}

RAS  127-11

PROCEDURE FOR DETERMINING THE MOMENT OF RESISTANCE AND

MINIMUM CHARACTERISTIC RESISTANCE LOAD TO INSTALL A TILE SYSTEM

ON A BUILDING OF A SPECIFIED ROOF SLOPE AND HEIGHT

Revise to read as follows:

1.            Scope

This standard covers the procedure for determining the Moment of Resistance (Mr) and Minimum Characteristic Resistance Load (F') to install a tile system on buildings of a specified roof slope and height. Compliance with the requirements and procedures herein specified, where the pressures (P asd) have been determined based on Table 1 or Table 2 of this standard,  as applicable, do not require additional signed and sealed engineering design calculation. All other calculations must be prepared, signed and sealed by a professional engineer or registered architect. Table 1 is applicable to a wind speed of 175 mph, risk category II buildings, and exposure category C. Table 2 is applicable to a wind speed of 175 mph, risk category II buildings, and exposure category D.

2.            How to determine the Moment Resistance (Mr) (Moment Based Systems)

2.1          Determine the minimum design wind pressures for the field, perimeter and corner areas (P asd 1, P asd 2 and P asd 3, respectively) using the values given in Table 1 or Table 2, as applicable,   or those obtained by engineering analysis prepared, signed and sealed by a professional engineer or registered architect based on ASCE 7.

2.2          Locate the aerodynamic multiplier (?) in tile Product Approval.

2.3          Determine the restoring moment due to gravity (Mg) per Product Approval.

2.4          Determine the attachment resistance (Mf) per Product Approval.

2.5          Determine the Moment of Resistance (Mr) per following formula:

Mr = (P asd x ?) - Mg

2.6          Compare the values for Mr, with the values for Mf, noted in the Product Approval. If the Mf values are greater than or equal to the Mr values, for each area of the roof [i.e., field P asd (1), perimeter P asd (2) and corner P asd (3) areas], then the tile attachment method is acceptable.

3.            How to determine the Minimum Characteristic Resistance Load (F') (Uplift Based System)

3.1          Determine the minimum design pressures for the field, perimeter and corner areas [P asd (1), Pasd (2) and P asd (3), respectively] using the values given in Table 1 or Table 2, as applicable,  or those obtained by engineering analysis prepared, signed and sealed by a professional engineer or registered architect based on the criteria set forth in ASCE 7.

3.2          Determine the angle (?) of roof slope, from Table 1 or Table 2, as applicable,.

3.3          Determine the length (l), width (w) and average tile weight (W) of tile, per Product Approval.

3.4          Determine the required uplift resistance (Fr) per following formula:

Fr = [(P asd x l x w) - W] x cos ?

3.5          Compare the values for Fr with the values for F' noted in the Product Approval. If the F' values are greater than or equal to the Fr values, for each area of roof [i.e., field P asd (1) perimeter (P asd (2) and corner P asd (3) areas], then the tile attachment method is acceptable.

| TABLE 1– Risk Category II Exposure Category 'C'1 MINIMUM DESIGN WIND UPLIFT PRESSURES IN PSF FOR FIELD (P asd (1), PERIMETER (P asd (2) AND CORNER (P asd (3) AREAS OF ROOFS FOR EXPOSURE C BUILDINGS WITH A ROOF MEAN HEIGHT AS SPECIFIED13 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ROOF SLOPE | > 2:12 to < 6:12 |  |  | > 6:12 to < 12:12 |  |
| Roof mean height | P asd (1) | P asd (2) | P asd (3)2 | P asd (1) | P asd (2) & P asd (3) |
| <=20' | -45.1 -39.1 | -78.6 -68.1 | -116.2 -100.7 | -49.1 -42.8 | -57.5 -50.0 |
| >20' to <=25' | -47.3 -40.9 | -82.3 -71.3 | -121.8 -105.4 | -51.2 -44.8 | -60.4 -52.3 |
| >25' to <=30' | -49.2 -42.4 | -85.7 -73.9 | -126.8 -109.3 | -53.6 -46.4 | -62.9 -54.3 |
| >30' to <=35' | -50.9 -43.9 | -88.5 -76.6 | -130.9 -113.2 | -55.3 -48.1 | -65.0 -56.2 |
| >35 to <=40' | -52.3 -45.1 | -91.0 -78.7 | -134.6 -116.3 | -56.9 -49.4 | -66.8 -57.8 |

1 Calculated in accordance with ASCE 7.05 (Wind Speed: 146 mph).

2 For Hip Roofs with slope <=5.5: 12, P asd (3) shall be treated as P asd (2). [Mod 2014r/2104r]

3 Pasd = 0.6Pult

| TABLE 2– Risk Category II Exposure Category 'D'1 MINIMUM DESIGN WIND UPLIFT PRESSURES IN PSF FOR FIELD (P asd (1), PERIMETER (P asd (2) AND CORNER (P asd (3) AREAS OF ROOFS FOR EXPOSURE D BUILDINGS WITH A ROOF MEAN HEIGHT AS SPECIFIED3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ROOF SLOPE | > 2:12 to < 6:12 |  |  | > 6:12 to < 12:12 |  |
| Roof mean height | P asd (1) | P asd (2) | P asd (3)2 | P asd (1) | P asd (2) & P asd (3) |
| <=20' | -47.0 | -81.9 | -121.0 | -51.4 | -60.1 |
| >20' to <=25' | -48.8 | -85.0 | -125.7 | -53.4 | -62.4 |
| >25' to <=30' | -50.3 | -87.7 | -129.6 | -55.0 | -64.4 |
| >30' to <=35' | -51.5 | -89.8 | -132.7 | -56.4 | -65.9 |
| >35 to <=40' | -52.7 | -91.9 | -135.8 | -57.7 | -67.9 |

1 Calculated in accordance with ASCE 7

2 For Hip Roofs with slope <=5.5: 12, P asd (3) shall be treated as P asd (2).

3 Pasd = 0.6Pult

| TABLE 23 WHERE TO OBTAIN INFORMATION |  |  |
| --- | --- | --- |
| Description | Symbol | Where to find |
| Design Pressure | Pasd (1) or Pasd (2) or Pasd (3) | Table 1 or Table 2, as applicable, or by an engineer analysis prepared, signed and sealed by a professional engineer based on ASCE 7 |
| Mean Roof Height | H | Job Site |
| Roof Slope | theta | Job Site |
| Aerodynamic Multiplier | lamda | Product Approval |
| Restoring Moment due to Gravity | Mg | Product Approval |
| Attachment Resistance | Mf | Product Approval |
| Required Moment Resistance | Mr | Calculated |
| Minimum Characteristic Resistance Load | F' | Product Approval |
| Required Uplift Resistance | Fr | Calculated |
| Average Tile Weight | W | Product Approval |
| Tile Dimensions | l=length w=width | Product Approval |

All calculations must be submitted to the building official at the time of permitting.

{R4800}

RAS 128-11

STANDARD PROCEDURE FOR DETERMINING APPLICABLE WIND DESIGN PRESSURES

FOR LOW SLOPE ROOF

Revise to read as follows:

1.            Scope

1.1 This roofing application standard has been developed to provide a responsive method of complying with the requirements of Chapters 15 & 16 (High-Velocity Hurricane Zones) of the Florida Building Code, Building . Compliance with the requirements and procedures herein specified, where the pressures (P asd) have been determined based on Table R128-1 1 or Table 2, of this standard, as applicable, do not require additional signed and sealed engineering design calculations. All other calculations must be prepared, signed and sealed by a professional engineer or registered architect.

2.            Definitions

2.1 For definitions of terms used in this application standard, refer to ASTM D 1079 and the Florida Building Code, Building.

3.            Applicability

3.1          This application standard applies to:

a.            exposure C and D category buildings; and

b.            building heights of less than or equal to 40 feet; and

c.             roof incline (pitch) is not greater than 1/2 in.:12 in.

d.            risk category II buildings

3.2          Using Table 1 or Table 2 below, as applicable,  determine the minimum design pressure for each respective roof area, which corresponds to the applicable roof height range.

| TABLE 1 – Risk Category II Exposure Category 'C'1 MINIMUM DESIGN WIND UPLIFT PRESSURES, IN PSF FOR FIELD (Pasd (1), PERIMETER (Pasd (2) AND CORNER (Pasd (3) AREAS OF ROOFS FOR EXPOSURE 'C' BUILDINGS2 |  |  |  |
| --- | --- | --- | --- |
| Roof mean height (below) | Pasd (1) Field | Pasd (2) (Perimeter) | Pasd (3) (Corners) |
| 20 | -49.2 -42.8 | -82.6 -71.7 | -124.3 -108.0 |
| 25 | -51.4 -44.8 | -86.3 -75.1 | -129.9 -113.0 |
| 30 | -53.6 -46.4 | -89.9 -77.8 | -135.3 -117.2 |
| 35 | -55.2 -48.1 | -92.7 -80.6 | -139.5 -121.3 |
| 40 | -56.9 -49.4 | -95.4 -82.9 | -143.6 -124.7 |

1 Calculated in accordance with ASCE 7.

2 Pasd = 0.6Pult

| TABLE 2– Risk Category II Exposure Category 'D' MINIMUM DESIGN WIND UPLIFT PRESSURES, IN PSF FOR FIELD (Pasd (1)), PERIMETER (Pasd (2)) AND CORNER (Pasd (3) AREAS OF ROOFS FOR EXPOSURE 'D' BUILDINGS |  |  |  |
| --- | --- | --- | --- |
| Roof mean height (below) | Pasd (1) Field | Pasd (2) (Perimeter) | Pasd (3) (Corners) |
| 20 | -51.4 | -86.2 | -129.7 |
| 25 | -53.4 | -89.5 | -134.7 |
| 30 | -55.0 | -92.3 | -138.9 |
| 35 | -56.4 | -94.5 | -142.3 |
| 40 | -57.7 | -96.8 | -145.6 |

1 Calculated in accordance with ASCE 7.

2 Pasd = 0.6Pult

3.3          Referencing the selected Roof Assembly Product Approval, check that the listed maximum allowable design pressure for the particular approved system meets or exceeds those listed in Table 1 or Table 2 above, as applicable.

{R4801}

TESTING APPLICATION STANDARD (TAS) No. 100-11 95

TEST PROCEDURE FOR WIND AND WIND DRIVEN RAIN RESISTANCE OF DISCONTINUOUS ROOF SYSTEMS

Revise Section 1.3 to read as follows:

1.3 All testing and calculations shall be conducted by an approved testing agency and all test reports, including calculations, shall be signed by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or Registered Roof Consultant.

Revise Section 8.4 to read as follows:

8.4 The test specimen shall be inspected by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or Registered Roof Consultant who shall confirm in the final report that the method of construction is in compliance with the specifications of this protocol. {3537}

Revise Section 11.1.2 to read as follows:

11.1.2 A detailed report of the method of construction, including a sketch of the test specimen; certification by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or Registered Roof Consultant that the test specimen was constructed in compliance with the specifications of this Protocol; and, a copy of the published application instructions provided by the prepared roof covering manufacturer.

{3537}

TESTING APPLICATION STANDARD (TAS) No. 100(A)-11 95

TEST PROCEDURE FOR WIND AND WIND DRIVEN RAIN RESISTANCE AND/OR INCREASED WINDSPEED RESISTANCE OF SOFFIT VENTILATION STRIP AND CONTINUOUS OR INTERMITTENT VENTILATION SYSTEM INSTALLED AT THE RIDGE AREA

Revise Section 10.4.8 to read as follows:

10.4.8 Manufacturers of Sstatic vents, turbines, powered vents or other protruding roof top components, having a height greater than 12 in. or any other dimension greater than 18 in. shall contact the authority having jurisdiction for additional testing requirements such as but not limited to be tested for resistance to wind induced pressures in compliance with TAS 100(B).

{3528}

TESTING APPLICATION STANDARD (TAS) No. 101-1195

TEST PROCEDURE FOR STATIC UPLIFT RESISTANCE OF MORTAR OR ADHESIVE SET

Revise Section 1.4 to read as follows:

1.4 All testing and calculations shall be conducted by an approved testing agency and all test reports, including calculations, shall be signed by a Registered Design Professional per F.S., Section 471 or 481Professional Engineer or Registered Roof Consultant. {3538}

Add 1.6 to read as follows:

1.6 For the purposes of the testing required in TAS 101-95, design pressures calculated in accordance with ASCE 7 are permitted to be multiplied by 0.6. (R4773)

Revise 4.4.2 to read as follows:

4.4.2 The Chief Code Compliance Officer Authority Having Jurisdiction  local building official shall determine the acceptability of system installation based on results from engineering design calculations in compliance with RAS 127. These calculations shall be: 1) performed for all buildings having a roof mean height greater than 40 feet; 2) performed, signed and sealed by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or Registered Architect; and, 3) shall take precedence over the procedures outlined in Section 4.4.1.  {3538, 4529}

Revise Section 7.2 to read as follows:

7.2	Underlayment

7.2.1 Underlayment shall be a standard 30/90 system with a 30 lb. ASTM D 226, Type II anchor sheet and an ASTM D 249 ASTM D 6380 CLASS M mineral surface roll roofing as the top ply.

7.2.2 The anchor sheet shall be mechanically attached to the wood sheathing with 12 ga. roofing nails and 15/8 in. 15/8 in. tin caps, in a 12 in. grid pattern staggered in two rows in the field and 6 in. o.c. attachment at any laps. The mineral surface top ply shall be applied in a full mopping of ASTM D 312, Type IV asphalt. {3539}

Revise Section 7.6.4 to read as follows:

7.6.4 All tile shall be applied with a minimum 3 in. head lap unless restricted by product design. {3539}

Revise Section 7.6.5 to read as follows:

7.6.5 The first course shall consist of two tiles installed at the lower edge of the test deck. The second course shall consist of the ‘test tile' installed over the first course, insuring for a minimum 3 in. head lap, unless restricted by product design. Tile in the first course which are not tested shall be installed to insure the "test tile," in the second course, is at the correct angle relative to the sheathing.  {3539}

Revise Section 7.8 to read as follows:

7.8 The test specimen shall be inspected by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or Registered Roof Consultant who shall certify, in the final test report, that it was constructed in compliance with the provisions of this Protocol. {3538}

Revise Section 10.3.3 to read as follows:

10.3.3 Determine the percent deviation from the mean (S'F) using the information from Section 10.3.2.1 10.3.2 and the following equation. {3539}

No change to the remaining text.

Revise Section 10.4 to read as follows:

10.4 Minimum Characteristic Resistance load (F’)

10.4.1 The mean ultimate load, F, used to determine the minimum characteristic resistance load (F’) shall be adjusted when the attachment system is used to provide resistance to more than one (1) roof tile by dividing the mean ultimate load by a load factor. For systems where the attachment provides resistance to a single roof tile the load factor shall be 1. For systems where the attachment provides resistance for two (2) roof tiles the load factor shall be 2. For systems where the attachment provides resistance for more than two (2) roof tiles the load factor shall be calculated and applied.

__            __

F =  ___  _F

Load Factor

10.4.1   10.4.2 Determine the minimum characteristic resistance load (F') using the mean ultimate load () and the following equation.

where, 

F' = minimum characteristic resistance load; 

F = mean ultimate load; 

MS = margin of safety = 2; and, 

W = average weight.

NOTE: The requirement noted in Section 10.3.3.1 shall be met prior to determining the minimum characteristic resistance load (F'). {4343}

Revise Section 11.1.3 to read as follows:

11.1.3 A detailed report of the method of test specimen construction, including a photograph of the test specimen and certification by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or Registered Roof Consultant that the test specimen was constructed in compliance with the specifications of this Protocol. {3538}

{R4534, R4529}

Testing Application Standards (TAS)102-11 95

Test Procedure for Static Uplift Resistance of Mechanically Attached‚

Rigid Roof Systems

Revise 1.4 to read as follows:

1.4 All testing and calculations shall be conducted by an approved testing agency and all test reports, including calculations, shall be signed by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or Registered Roof Consultant.

Revise 4.4.2 to read as follows:

4.4.2 The Authority Having Jurisdiction Chief Code Compliance Officer local building official shall determine the acceptability of system installation based on results from engineering design calculations in compliance with RAS 127. These calculations shall be: 1) performed for all buildings having a roof mean height greater than 40 feet; 2) performed, signed and sealed by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or a Registered Architect; and, 3) shall take precedence over the procedures outlined in Section 4.4.1.

Revise 7.2.1 to read as follows:

7.2.1 Underlayment shall be a standard 30/90 system with a 30 lb ASTM D 226, type II anchor sheet and an ASTM D 249 ASTM D 6380 CLASS M mineral surface roll roofing as the top ply.

Revise 7.8 to read as follows:

7.8 The test specimen shall be inspected by a Registered Design Professional per F.S., Section 471 or 481  Professional Engineer or Registered Roof Consultant who shall certify, in the final test report, that it was constructed in compliance with the provisions of this Protocol.

Revise 10.1.3 to read as follows:

10.1.3  A detailed report of the method of test specimen construction, including a photograph of the test specimen and certification by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or Registered Roof Consultant that the test specimen was constructed in compliance with the specifications of the Protocol.

{3540, 4535, 4536}

ROOFING APPLICATION STANDARD (TAS) No. 102(A)-1195

TEST PROCEDURE FOR STATIC UPLIFT RESISTANCE OF MECHANICALLY ATTACHED, CLIPPED, RIGID, ROOF SYSTEMS

Revise Section 1.4 to read as follows:

1.4 All testing and calculations shall be conducted by an approved testing agency and all test reports, including calculations, shall be signed by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or Registered Roof Consultant. {3541}

Revise Section 4.4.2 to read as follows:

4.4.2 The Authority Having Jurisdiction Chief Code Compliance Officer local building official shall determine the acceptability of system installation based on results from engineering design calculations in compliance with RAS 127. These calculations shall be: 1) performed for all buildings having a roof mean height greater than 40 feet; 2) performed, signed and sealed by a Professional Engineer or a Registered Architect; and, 3) shall take precedence over the procedures outlined in Section 4.4.1. {3541}

Revise Section 7.2.1 to read as follows:

7.2.1 Underlayment shall be a standard 30/90 system with a 30 lb. ASTM D 226, type II anchor sheet and an ASTM D 249 ASTM D 6380 CLASS M mineral surface roll roofing as the top ply. {3541}

Revise Section 7.8 to read as follows:

7.8 The test specimen shall be inspected by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or Registered Roof Consultant who shall certify, in the final test report, that it was constructed in compliance with the provisions of this Protocol. {3541}

Revise Section 10.1.4 to read as follows:

10.1.4 A detailed report of the method of test specimen construction, including a photograph of the test specimen and certification by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or Registered Roof Consultant that the test specimen was constructed in compliance with the specifications of the Protocol.

{3541}

ROOFING APPLICATION STANDARD (TAS) No. 103-1195

TEST PROCEDURE FOR SELF-ADHERED UNDERLAYMENTS FOR USE IN DISCONTINUOUS ROOF SYSTEMS

Revise 7.1.1.2 to read as follows:

7.1.1.2 Four (4) 8 in.' x 8' test decks shall be constructed of 40/20 19/32 in. APA Rated Sheathing attached to wood joists spaced 24 o.c. Each test deck shall consist of four (4) panels of said sheathing, the corners of which shall meet at the center of each test deck, leaving a 1/8 in. gap between panels.

(R4788)

Revise Section 22 to read as follows:

FOR MINERAL SURFACED ROLL MATERIAL TO BE 
USED AS A MORTAR OR ADHESIVE SET TILE UNDERLAYMENT

22. Granule Adhesion 

22.1 This test covers the determination of granule loss of materials specified in Section 1 of this Protocol, which employ a granular surfacing on one side, in accordance with ASTM Test Method D 5147, Section 14, except as noted below.

22.1.1 Any test specimen which exhibits an average granule loss greater than 0.75 grams shall be considered as failing the granule adhesion test. {3628}

TESTING APPLICATION STANDARD (TAS) No. 104-1195

TEST PROCEDURE FOR NAIL-ON UNDERLAYMENT

FOR USE IN DISCONTINUOUS ROOF SYSTEMS

Revise Section 18 as follows:

FOR MINERAL SURFACED ROLL MATERIALS TO BE USED

AS A MORTAR OR ADHESIVE SET TILE UNDERLAYMENT

18. Granule Adhesion 

18.1 This test covers the determination of granule loss of materials specified in Section 1 of this Protocol, which employ a granular surfacing on one side, in accordance with ASTM Test Method D 5147, Section 14, except as noted below.

18.1.1 Any test specimen which exhibits an average granule loss greater than 0.75 grams shall be considered as failing the granule adhesion test.

{3630}

TAS 105-11 98

TEST PROCEDURE FOR FIELD WITHDRAWAL RESISTANCE TESTING

Revise 1.3 to read as follows:

1.3 For the purposes of the testing required in TAS 105-11, design pressures calculated in accordance with ASCE 7 are permitted to be multiplied by 0.6.

{R4775}

TESTING APPLICATION STANDARD (TAS) No. 106-11

STANDARD PROCEDURE FOR FIELD VERIFICATION OF THE BONDING OF MORTAR

OR ADHESIVE SET TILE SYSTEMS

AND MECHANICALLY ATTACHED, RIGID, DISCONTINUOUS ROOF SYSTEMS

Revise Section 4.1.2 to read as follows:

4.1.2 If more than 3% of the tile in any roof area arc are determined to be loose. No further testing shall be conducted and the Roof System shall be considered to have failed the testing and be unacceptable.

{3527}

TAS 107-11

TEST PROCEDURE FOR WIND RESISTANCE TESTING OF NON-RIGID,

DISCONTINUOUS ROOF SYSTEM ASSEMBLIES

Revise 11.1 to read as follows:

11.1 A test report will be provided to the Chief Code Compliance Officer Authority Having Jurisdiction confirming successful compliance with the test provisions of this Protocol. Completion of this test Protocol is one in a series of Testing Application Standards required by the Florida Building Code, Building for Product Approval of non-rigid, discontinuous Roof System Assemblies.

{R4537}

TESTING APPLICATION STANDARD (TAS) No. 108-1195

TEST PROCEDURE FOR WIND TUNNEL TESTING OF AIR PERMEABLE, RIGID, DISCONTINUOUS ROOF SYSTEMS

Revise Section 4.1 to read as follows:

4.1 Use of this protocol is limited to systems which are air permeable, as determined in compliance with TAS 116 or deemed air permeable by the Authority Having Jurisdiction Chief Code Compliance Officer local building official, in which the unsealed, overlapping, rigid components have a length between 1.0 and 1.75 feet; an exposed width between 0.73 and 1.25 feet; and a thickness not greater than 1.3 in.

Note: The Authority Having Jurisdiction Chief Code Compliance Officer local building official may waive the requirement for air permeability testing in compliance with TAS 116.

{3545, 4538}

Revise Section 7.2.1 to read as follows:

7.2.1 The underlayment shall be a standard 30/90 system with a 30 ASTM D 226, type II anchor sheet and an ASTM D 249 ASTM D 6380 CLASS M mineral surface roll roofing as the top ply. {3545}

Revise Section 7.11 to read as follows:

7.11 The test specimen shall be inspected by a Registered Design Professional per F.S., Section 471 or 481 professional engineer or Registered Roof Consultant, who shall certify, in the final report, that it was constructed in compliance with the provisions of this protocol. {3545}

Revise Section 11.1.2 to read as follows:

11.1.2 A detailed report of the test specimen construction, including a photograph of the test specimen and certification by a Registered Design Professional per F.S., Section 471 or 481 professional engineer or Registered Roof Consultant that the test specimen was constructed in compliance with the specifications of this Protocol. {3545}

TESTING APPLICATION STANDARD (TAS) No. 110-112000

TESTING REQUIREMENTS FOR PHYSICAL PROPERTIES OF ROOF MEMBRANES, INSULATION, COATINGS, AND OTHER ROOFING COMPONENTS

Revise Section 2.2 to read as follows:

2.2	Roofing components within asphalt built-up and modified bitumen roof assemblies shall be in compliance with the following requirements, as applicable.

PRODUCT TEST STANDARD

Membrane or Roll Roofing Products

Roll Roofing, Organic, Granule Surface ASTM D 249 ASTM D 6380 Class M

Remaining text in the chart above remained unchanged.

{3546}

TESTING APPLICATION STANDARD (TAS) No. 111(A)-1195

TEST PROCEDURE FOR ROOF EDGE TERMINATION PERFORMANCE

Revise Section 1.3 to read as follows:

1.3 All testing and calculations shall be conducted by an approved testing agency and all test reports, including calculations, shall be signed by a Registered Design Professional per F.S., Section 471 or 481 professional engineer or Registered Roof Consultant.

{3547}

TESTING APPLICATION STANDARD (TAS) No. 111(B)-1195

TEST PROCEDURE FOR EDGE METAL PULL-OFF PERFORMANCE

Revise Section 1.3 to read as follows:

1.3 All testing and calculations shall be conducted by an approved testing agency and all test reports, including calculations, shall be signed by a professional engineer Registered Design Professional per F.S., Section 471 or 481 or Registered Roof Consultant.

{3548}

TESTING APPLICATION STANDARD (TAS) No. 111(C)-1195

TEST PROCEDURE FOR COPING CAP PULL-OFF PERFORMANCE

Revise Section 1.3 to read as follows:

1.3 All testing and calculations shall be conducted by an approved testing agency and all test reports, including calculations, shall be signed by a Registered Design Professional per F.S., Section 471 or 481 professional engineer or Registered Roof Consultant. {3549}

Revise Section 2.4 to read as follows:

2.4 Single Ply Roofing Institute: 
ANSI/SPRI ES-1 Edge Design Guide, March 1994

{3549}

Chapter No. 112-1195

Standard Requirements for Concrete Roof Tiles

Revise 11.1 to read as follows:

11.1 The Authority Having Jurisdiction Chief Code Compliance Officer local building official shall certify that the tile is in compliance with this protocol by the issuance of a Product Approval to the applicant. Tile forms part of a system that requires testing under additional protocols for usage in the high- velocity hurricane zone jurisdiction.

{4539}

Appendix A - Clarification of Testing Frequency and Sampling Method

Revise 3.4 to read as follows:

3.4 Testing shall be conducted quarterly. All sampling, testing and calculations shall be conducted by an approved testing agency. All test results, including calculations, shall be signed by a Registered Design Professional per F.S., Section 471 or 481 professional engineer or Registered Roof Consultant.

TESTING APPLICATION STANDARD (TAS) No. 114-11 95

TEST PROCEDURES FOR ROOF SYSTEM ASSEMBLIES IN THE HIGH-VELOCITY HURRICANE ZONE JURISDICTION

Add 1.9 to read as follows:

1.9 Design pressures calculated in accordance with ASCE 7 are permitted to be multiplied by 0.6 for the purposes of comparing to tested pressures in TAS 114-11.

{R4776}

Revise 4.4.1 to read as follows:

4.4.1      The effective date of a protocol mandates that all products tested for Approval after the effective date must satisfy the requirements of that protocol. Products Approved under a previous Protocol must comply with the new version by the effective date or such date established by the chief code compliance officer Authority Having Jurisdiction or else forfeit Product Approval. The effective date may apply to the entire protocol, or, where so indicated, only to specific paragraphs of the protocol.

Revise Section 5, Test Specimens , to read as follows:

5.         Test Specimens:

5.1       The components for a proposed test panel are assembled to the desired specifications and details (gauge of steel, application method and rate for the adhesives, size and thickness of insulation, type of cover) and then left to "cure" for a specified time period.

5.2       The test specimen shall be tested to 30 psf (1.4 kPa) after a 4 day laboratory cure time at ambient conditions. If the test specimen fails to resist this initial test pressure, the test shall be discontinued. On passing this initial testing, the test specimen shall be allowed to cure for the remaining cure time.

5.3       If insulation panels for part of the test specimen, a panels shall be installed such that a three way joint is located in the center of the test specimen. If more than one layer of insulation forms part of the test specimen, the top layer shall employ the three way joint.

5.4       Roof system assemblies whose wind-load resistance performance may be affected by bad weather conditions during installation shall be constructed in a manner which simulates actual working conditions.

5.5       Not less than three test specimens shall be constructed for each roof system assembly being tested.

6.         Test Procedure:

6.1       Principal

6.1.1   The test apparatus is secured to the roof system assembly test specimen which is cut around the perimeter of the test apparatus. Thereafter, an uplift load is applied to the test apparatus which distributes the load over its area. The distributed load is transferred to the test specimen. Subsequent increasing uplift loads are applied until failure occurs.

6.2       Once the test specimen has cured and the test apparatus is secured, uplift loads are applied through the test apparatus in accordance with Table D1, below:

6.3       Prior to and during the attainment of the uplift pressures noted above, the test specimen is examined for failure. On failure, the test specimen is dismantled and examined to determine the exact mode of failure.

6.4       Record the mode, time, and pressure interval of failure.

7.         Interpretation of Results:

7.1       The passing uplift pressure shall be the pressure which the test specimen resisted for one minute without failure. The passing uplift pressure shall be the average of the three pressures which the test specimens resisted for one minute without failure. If one or more of the three tests yields a passing uplift pressure greater or less than 15 percent of other recorded values, an additional test shall be conducted.

7.2       The minimum passing uplift pressure for an approved roof system assembly shall be 90 psf (4.2 kPa).

7.3       A 2:1 margin of safety shall be applied to the passing uplift pressure prior to inclusion in the system manufacturer's Product Approval.

7.4       Average wind velocities can vary considerably from area to area. The Florida Building Code, Building utilizes a windspeed as noted in section 1620.2. These wind velocities in miles per hour are related to the design pressure, in pounds per square feet (kg/m2), for a particular building. Refer to Chapter 16 (High-Velocity Hurricane Zones) of the Florida Building Code, Building and ASCE 7.

7.5       No extrapolation of resulting data will be accepted. (3532A1)

Revise to add 7.6 as new section:

7.6 Design pressures calculated in accordance with ASCE 7 are permitted to be multiplied by 0.6 for the purposes of comparing to tested pressures in TAS 114-11.

Revise 8.1.1 to read as follows:

8.1.1      This protocol is intended to evaluate a roof system assembly for its performance as it relates to accelerated weathering, corrosion of metal parts, F.I.T. performance (Fatigue, Indentation and Temperature), fire, foot traffic, hail, leakage, and wind. The applicant may submit up to five roof system assemblies in its Application for Product Approval. The chief code compliance officer Authority Having Jurisdiction shall respond, in writing, stating which tests shall be required for sufficient evidence of compliance.

Revise 8.2.3 to read as follows:

8.2.3      Tests of alternate constructions may be waived by the chief code compliance officer Authority Having Jurisdiction if considered less hazardous than those previously tested.

8.3.2.3   Simulated uplift testing:

*                                                             *                                                             *

The chief code compliance officer Authority Having Jurisdiction may, at his/her option, accept and publish in Product Approvals roof system assemblies that do not meet the minimum 90 psf (4.2 kPa) uplift criteria providing the manufacturer has one or more assemblies that meet the minimum uplift criteria.

Revise 9.2.1.3 to read as follows:

9.2.1.3   The chief code compliance officer Authority Having Jurisdiction may rely on continued listing of approved systems in the annual Factory Mutual Approval Guide and quarterly supplements as confirmation of requirements under this section.

Revise 9.2.2.2 to read as follows:

9.2.2.2   The chief code compliance officer Authority Having Jurisdiction shall review established procedures from recognized testing and listing agencies to confirm compliance with the requests set forth herein.

{R4540, R4777}

APPENDIX A  TEST PROCEDURE FOR ABOVE DECK COMBUSTIBILITY

ASTM E 108

1.2  - revise to read as follows:

1.2          The applicant shall submit in the initial application all roof system assemblies requested over combustible and noncombustible decks. From this submission, the chief code compliance officer Authority Having Jurisdiction may list a limited number of roof system assemblies which will require external fire testing to represent all proposed assemblies.

{F4541}

TESTING APPLICATION STANDARD (TAS) 116-1195

TEST PROCEDURE FOR AIR PERMEABILITY TESTING OF RIGID,

DISCONTINUOUS ROOF SYSTEMS

Revise Section 1.4 to read as follows:

1.4 All testing and calculations shall be conducted by an approved testing agency and all test reports, including calculations, shall be signed by a Registered Design Professional per F.S., Section 471 or 481 professional engineer or Registered Roof Consultant. {3553}

Revise Section 4.2 to read as follows:

4.2 The procedures and calculations outlined herein provide a means of determining the air permeability (Cd) for a rigid, discontinuous roof system. This value shall be listed in the system manufacturer's roof system assembly Product Approval for reference by the chief code compliance officer to determine whether the system is air permeable or air impermeable. The Authority Having Jurisdiction Chief Code Compliance Officer local building official may nullify this testing requirement for systems which are generally considered air permeable. {3553, 4542}

Revise Section 4.3 to read as follows:

4.3 The test criteria will be based on testing, under the provisions of this Protocol, of rigid, prepared roof coverings which are generally considered air impermeable (i.e. metal roof systems, and lap sealed, rigid prepared roof coverings). This criteria shall be determined by September 1, 1994. Until that date, the Authority Having Jurisdiction Chief Code Compliance Officer local building official shall judge as to whether a prepared roof covering is air permeable or air impermeable. {3553, 4542}

Revise Section 12.1.2 to read as follows:

12.1.2 A detailed report of the system installation prepared by a Registered Design Professional per F.S., Section 471 or 481 professional engineer or Registered Roof Consultant, including a sketch of the installed system, indicating the sealed and unsealed components, and the published installation instructions provided by the system manufacturer.

{3553}

TESTING APPLICATION STANDARD (TAS) 117(A)-1195

TEST PROCEDURE FOR WITHDRAWAL RESISTANCE TESTING OF MECHANICAL FASTENERS USED IN ROOF SYSTEM ASSEMBLIES

1.3 All testing and calculations shall be conducted by an approved testing agency and all test reports, including calculations, shall be signed by a Registered Design Professional per F.S., Section 471 or 481 professional engineer or Registered Roof Consultant.

{3554}

TESTING APPLICATION STANDARD (TAS) 117(B)- 1195

TEST PROCEDURE FOR DYNAMIC PULL-THROUGH PERFORMANCE OF ROOFING MEMBRANES OVER FASTENER HEADS OR FASTENERS WITH METAL BEARING PLATES

Revise Section 1.4 to read as follows:

1.4 All testing and calculations shall be conducted by a certified testing agency and all test reports, including calculations, shall be signed and sealed by a Registered Design Professional per F.S., Section 471 or 481 professional engineer or Registered Roof Consultant.

{3555}

TESTING APPLICATION STANDARD (TAS) 117(C)-1195

TEST PROCEDURE FOR DYNAMIC PULL-OFF PERFORMANCE OF ROOFING FASTENER HEADS OR FASTENERS WITH BEARING PLATES

Revise Section 1.4 to read as follows:

1.4 All testing and calculations shall be conducted by a certified testing agency and all test reports, including calculations, shall be signed and sealed by a Registered Design Professional per F.S., Section 471 or 481 professional engineer or Registered Roof Consultant.

{3556}

TAS 121-11

STANDARD REQUIREMENTS FOR TESTING AND APPROVAL

OF ROOFING ADHESIVES, MASTICS AND COATINGS

Revise 8.1 to read as follows:

8.1 The authority having jurisdiction may periodically purchase commercial quantities of the approved product for testing at approved Testing Agencies to confirm compliance with the provisions of the ASTM Standard or Testing Application Standards. Failure to meet the minimum physical property and performance properties of the test standard shall constitute grounds for rejection of the lots and suspension of the Product Approval. In cases of rejection the chief code compliance officer Authority Having Jurisdiction shall request removal of the rejected lot number(s) from commercial sale.

Revise 8.2 to read as follows:

8.2 The chief code compliance officer Authority Having Jurisdiction may, after rejection of one or more lots, require third party quality control inspection as a provision to lifting of approval suspension.

{R4543}

TAS 123-11

STANDARD REQUIREMENTS FOR MORTAR

USED IN MORTAR SET TILE SYSTEMS

Revise 10.1 to read as follows:

10.1 The chief code compliance officer Authority Having Jurisdiction may periodically purchase commercial quantities of the Approved product for testing to confirm compliance with the provisions of this protocol. Failure to meet the minimum requirements set forth in this protocol shall constitute grounds for rejection of the lots and suspension of the Product Approval. In cases of rejection the chief code compliance officer Authority Having Jurisdiction shall request removal of the rejected lot number(s) from commercial sale.

{R4544}

TESTING APPLICATION STANDARD (TAS) 124-11 95

TEST PROCEDURE FOR FIELD UPLIFT RESISTANCE OF EXISTING MEMBRANE ROOF SYSTEMS AND IN SITU TESTING FOR REROOF AND NEW CONSTRUCTION APPLICATIONS

Revise Section 1.6 to read as follows:

1.6 All testing shall be conducted by an approved testing agency and all test reports shall be signed by a Registered Design Professional per F.S., Section 471 or 481 professional engineer or Registered Roof Consultant.  {3560}

Revise 1.7 to read as follows:

1.7 Design pressures calculated in accordance with ASCE 7 are permitted to be multiplied by 0.6 for the purposes of comparing to tested pressures in TAS 124-11.  (R4778)

TESTING APPLICATION STANDARD (TAS) 126-1195

STANDARD PROCEDURES FOR ROOF MOISTURE SURVEYS

Revise Section 1.3 to read as follows:

1.3 All testing and calculations shall be conducted by an approved testing agency and all test reports, including calculations, shall be signed by a Registered Design Professional per F.S., Section 471 or 481 Professional Engineer or a Registered Roof Consultant.

{3561}

TAS 131-11

STANDARD REQUIREMENTS FOR THERMOPLASTIC OLEFIN

ELASTOMER BASED SHEET USED IN SINGLE-PLY ROOF MEMBRANE

Revise 11.2 to read as follows:

11.2 If the results of any tests do not conform to the requirements of this specification, retesting to determine conformity shall be performed as required by the Chief Code Compliance Officer Authority Having Jurisdiction.

Revise 14.1 to read as follows:

14.1 Upon request of the Chief Code Compliance Officer Authority Having Jurisdiction, a manufacturer may be required to certify that the material was manufactured and tested in accordance with this Protocol. Additional testing for confirmation may be required by an approved testing agency.

{R4545}

TAS 132-11

STANDARD REQUIREMENTS FOR TESTING AND APPROVAL

OF SEALANTS USED IN ROOFING

Revise 10 to read as follows:

10.          Rejection and Reinspection

10.1 The Building Code Compliance Office Authority Having Jurisdiction may periodically purchase commercial quantities of the approved product for testing at Approved Testing Agencies to confirm compliance with the provisions of this Protocol. Failure to meet the minimum requirements set forth in Section 5 shall constitute grounds for rejection of the lots and suspension of the Product Approval. In cases of rejection the Authority Having Jurisdiction shall request removal of the rejected lot number(s) from commercial sale.

10.2 The Chief Code Compliance Officer Authority Having Jurisdiction may, after rejection of one or more lots, require third party quality control inspection as a provision to lifting of Approval suspension.  (R4546)

TAS 135-11

STANDARD REQUIREMENTS FOR FIBERGLASS REINFORCED TILE,

SHINGLES OR PANELS AND FIBER CEMENT SHINGLES, SHAKES OR PANELS

Revise 1.2 to read as follows:

1.2 The designation of fiberglass reinforced composite products between tile, shingles or panels, and the corresponding test requirements thereof, shall be made by the Chief Code Compliance Officer Authority Having Jurisdiction.

Revise 1.3 to read as follows:

1.3 The designation of non-asbestos, fiber cement products between shingles, shakes or panels, and the corresponding test requirements thereof, shall be made by the Chief Code Compliance Officer Authority Having Jurisdiction.

{R4548}

TAS 138-11

STANDARD REQUIREMENTS FOR ALUMINUM PIGMENTED EMULSIFIED ASPHALT

USED AS A PROTECTIVE COATING FOR ROOFING

Revise 11.1 and 11.2 to read as follows:

11.1 The Authority Having Jurisdiction may periodically purchase commercial quantities of the approved product for testing at Approved Testing Agencies to confirm compliance with the provisions of this Protocol. Failure to meet the minimum requirements set forth in Sections 6 and 7 shall constitute grounds for rejection of the lots and suspension of the Product Approval. In cases of rejection the Chief Code Compliance Officer Authority Having Jurisdiction shall request removal of the rejected lot number(s) from commercial sale.

11.2 The Chief Code Compliance Officer Authority Having Jurisdiction may, after rejection of one or more lots, require third party quality control inspection as a provision to lifting of Approval suspension.

{R4549}

TAS 139-11

STANDARD REQUIREMENTS FOR WHITE ROOF PATCH SPECIFICATION

Revise 9.1 and 9.2 to read as follows:

9.1 The Authority Having Jurisdiction may periodically purchase commercial quantities of the approved product for testing at Approved Testing Agencies to confirm compliance with the provisions of this Protocol. Failure to meet the minimum requirements set forth in Sections 5 and 6 shall constitute grounds for rejection of the lots and suspension of the Product Approval. In cases of rejection the Chief Code Compliance Officer Authority Having Jurisdiction shall request removal of the rejected lot number(s) from commercial sale.

9.2 The Chief Code Compliance Officer Authority Having Jurisdiction may, after rejection of one or more lots, require third party quality control inspection as a provision to lifting of Approval suspension.

{R4566}

TAS 140-11 STANDARD REQUIREMENTS FOR NONFIBERED ROOF AND FOUNDATION COATINGS Revise 8.1 and 8.2 to read as follows: 8.1 The Authority Having Jurisdiction may periodically purchase commercial quantities of the approved product for testing at Approved Testing Agencies to confirm compliance with the provisions of this Protocol. Failure to meet the minimum requirements set forth in Sections 5 and 6 shall constitute grounds for rejection of the lots and suspension of the Product Approval. In cases of rejection the Chief Code Compliance Officer Authority Having Jurisdiction shall request removal of the rejected lot number(s) from commercial sale. 8.2 The Chief Code Compliance Officer Authority Having Jurisdiction may, after rejection of one or more lots, require third party quality control inspection as a provision to lifting of Approval suspension.

{R4567}

TAS 141-11

STANDARD REQUIREMENTS FOR COAL TAR (CUTBACK) ROOF COATING,

BRUSHING CONSISTENCY

Revise 10.1 and 10.2 to read as follows:

10.1 The Authority Having Jurisdiction may periodically purchase commercial quantities of the approved product for testing at Approved Testing Agencies to confirm compliance with the provisions of this Protocol. Failure to meet the minimum requirements set forth in Sections 5 and 6 shall constitute grounds for rejection of the lots and suspension of the Product Approval. In cases of rejection the Chief Code Compliance Officer Authority Having Jurisdiction shall request removal of the rejected lot number(s) from commercial sale.

10.2 The Chief Code Compliance Officer Authority Having Jurisdiction may, after rejection of one or more lots, require third party quality control inspection as a provision to lifting of Approval suspension.

{R4568}

TAS 142-11

STANDARD REQUIREMENTS FOR COAL TAR ROOF CEMENT, ASBESTOS FREE

Revise 9.1 and 9.2 to read as follows:

9.1 The Authority Having Jurisdiction may periodically purchase commercial quantities of the approved product for testing at Approved Testing Agencies to confirm compliance with the provisions of this Protocol. Failure to meet the minimum requirements set forth  in Sections 5 and 6 shall constitute grounds for rejection of the lots and suspension of the Product Approval. In cases of rejection the Chief Code Compliance Officer Authority Having Jurisdiction shall request removal of the rejected lot number(s) from commercial sale.

9.2 The Chief Code Compliance Officer Authority Having Jurisdiction may, after rejection of one or more lots, require third party quality control inspection as a provision to lifting of Approval suspension.

{R4569}

TAS 143-11

STANDARD REQUIREMENTS FOR WHITE ELASTOMERIC ROOF COATINGS

USED FOR COATING BUILT UP ROOFS AND METAL ROOFING SYSTEMS

Revise 9.1 and 9.2 to read as follows:

9.1 The Authority Having Jurisdiction may periodically purchase commercial quantities of the approved product for testing at Approved Testing Agencies to confirm compliance with the provisions of this Protocol. Failure to meet the minimum requirements set forth in Sections 5 and 6 shall constitute grounds for rejection of the lots and suspension of the Product Approval. In cases of rejection the Chief Code Compliance Officer Authority Having Jurisdiction shall request removal of the rejected lot number(s) from commercial sale.

9.2 The Chief Code Compliance Officer Authority Having Jurisdiction may, after rejection of one or more lots, require third party quality control inspection as a provision to lifting of Approval suspension.

{R4571}

TAS 201-11

IMPACT TEST PROCEDURES

Revise 13.2.1 to read as follows: 13.2.1 Permanent label on storm panels could be printed directly on each panel at intervals not to exceed 36 24 inches with non removable paint or ink.

{S4505}

TESTING APPLICATION STANDARD (TAS) 202-11 94

CRITERIA FOR TESTING IMPACT & NONIMPACT RESISTANT BUILDING ENVELOPE COMPONENTS USING UNIFORM STATIC AIR PRESSURE

Revise 1.1 to read as follows:

1.1 This protocol covers procedures for conducting a uniform static air pressure test for materials and products such as wall cladding, glass block, exterior doors, garage doors, skylights, exterior windows, storm shutters, and any other external component which help maintain the integrity of the building envelope.

For the purposes of the testing required in TAS 202 Section 5.2, design pressures calculated in accordance with ASCE 7 are permitted to be multiplied by 0.6.

{S4779}

Revise 5.2.2.2 to read as follows: 5.2.2.2 In the case of doors and windows that are not required to comply with the means of egress/escape requirements which are provided with more than one single action hardware and comply with the test described in this protocol, shall also be successfully tested with a test load equal to a static air pressure based on wind velocity of 97 75 mph using only one single action locking mechanism. Test pressures are permitted to multiplied by 0.6 as specified in Section 1.1. (remainder of text unchanged).

{S4780}

Revise Section 5.2.7 to read as follows:

5.2.7 Air Infiltration - Shall comply with ASTM E283-91.

{3464}

Revise 13.2.1 to read as follows:

13.2.1 Permanent label on storm panels could be printed directly on each panel at intervals not to exceed 36 24 inches with non removable paint or ink.

{4784}

TAS – 203 CRITERIA FOR TESTING PRODUCTS SUBJECT TO

CYCLIC WIND PRESSURE LOADING

Revise 14.2.1 to read as follows:

14.2.1 Permanent label on storm panels could be printed directly on each panel at intervals not to exceed 36 24 inches with non removable paint or ink.

(S4785)

TAS 301  TESTING LABORATORY

Revise 8.1 to read as follows:

8.            Termination of Certification and Registration of Testing Facility:

8.1 At any time the facility may voluntarily terminate its certification and registration with the Authority Having Jurisdiction, either in its entirety or with respect to any particular test, simply by giving written notification to the Building Code Compliance Office Authority Having Jurisdiction. Such written notification shall state the date as of which the termination is to take place. The Building Code Compliance Office Authority Having Jurisdiction shall remove the name of the facility from the approved list.

(S4572)