LOCATION OF FLORIDA LAW TEXT IN 2007 FACBC AND DRAFT 2012 FACBC

See end of this file for explanation of text highlighting and location in documents.

PART II

ACCESSIBILITY BY HANDICAPPED PERSONS

  - Short title.--Sections 553.501-553.513 may be cited as the "Florida Americans With Disabilities Accessibility Implementation Act."

553.502  Intent.--The purpose and intent of ss. 553.501-553.513 is to incorporate into the law of this state the accessibility requirements of the Americans with Disabilities Act of 1990, Pub. L. No. 101-336, 42 U.S.C. ss. 12101 et seq., and to obtain and maintain United States Department of Justice certification of the Florida Accessibility Code for Building Construction as equivalent to federal standards for accessibility of buildings, structures, and facilities. All state laws, rules, standards, and codes governing facilities covered by the guidelines shall be maintained to assure certification of the state's construction standards and codes. Nothing in ss. 553.501-553.513 is intended to expand or diminish the defenses available to a place of public accommodation under the Americans with Disabilities Act and the federal Americans with Disabilities Act Accessibility Guidelines, including, but not limited to, the readily achievable standard, and the standards applicable to alterations to places of public accommodation.

History.--s. 1, ch. 93-183; s. 1, ch. 97-76.

FACBC 1.2

Draft 2012 FACBC section 101.1.3.1 and 101.3

553.503  Adoption of guidelines.--Subject to the exceptions in s. 553.504, the federal Americans with Disabilities Act Accessibility Guidelines, as adopted by reference in 28 C.F.R., part 36, subparts A and D, and Title II of Pub. L. No. 101-336, are hereby adopted and incorporated by reference as the law of this state. The guidelines shall establish the minimum standards for the accessibility of buildings and facilities built or altered within this state. The 1997 Florida Accessibility Code for Building Construction must be adopted by the Florida Building Commission in accordance with chapter 120.

History.--s. 1, ch. 93-183; s. 2, ch. 97-76; s. 65, ch. 2000-141; s. 59, ch. 2000-154.

NOT A PART OF FACBC

553.504  Exceptions to applicability of the guidelines.--Notwithstanding the adoption of the Americans with Disabilities Act Accessibility Guidelines in s. 553.503, all buildings, structures, and facilities in this state shall meet the following additional requirements when they provide increased accessibility:

(1)  All new or altered buildings and facilities subject to ss. 553.501-553.513 which may be frequented in, lived in, or worked in by the public shall comply with ss. 553.501-553.513.

FACBC 1.2

Draft 2012 FACBC section 101.1.2

(2)  All new single-family houses, duplexes, triplexes, condominiums, and townhouses shall provide at least one bathroom, located with maximum possible privacy, where bathrooms are provided on habitable grade levels, with a door that has a 29-inch clear opening. However, if only a toilet room is provided at grade level, such toilet room shall have a clear opening of not less than 29 inches.

FACBC 1.3 [refers to 553.504(2) but does not include its text] and FACBC 4.1.1(1)(c) and 4.22.2 and 4.23.2 and 11.1(1)

Draft 2012 FACBC section 233.3.6

(3)  All required doors and walk-through openings in buildings excluding single-family homes, duplexes, and triplexes not covered by the Americans with Disabilities Act of 1990 or the Fair Housing Act shall have at least 29 inches of clear width except under ss. 553.501-553.513.

FACBC 4.13.1(2)

Draft 2012 FACBC section 404.2.3

(4)  In addition to the requirements in reference 4.8.4 of the guidelines, all landings on ramps shall be not less than 60 inches clear, and the bottom of each ramp shall have not less than 72 inches of straight and level clearance.

FACBC 4.8.4(2)

Draft 2012 FACBC yellow shaded text in section 405.7.3 and blue shaded text is similar to ADAAG language so did not modify ADAAG

(5)  All curb ramps shall be designed and constructed in accordance with the following requirements:

(a)  Notwithstanding the requirements of reference 4.8.5.2 of the guidelines, handrails on ramps which are not continuous shall extend not less than 18 inches beyond the sloped segment at both the top and bottom, and shall be parallel to the floor or ground surface.

FACBC 4.8.5(2)

Draft 2012 FACBC section 505.10.1

(b)  Notwithstanding the requirements of references 4.3.3 and 4.8.3 of the guidelines, curb ramps that are part of a required means of egress shall be not less than 44 inches wide.

FACBC 4.3.3 and 4.8.3

Draft 2012 FACBC section 406.1.1

(c)  Notwithstanding the requirements of reference 4.7.5 of the guidelines, curb ramps located where pedestrians must use them and all curb ramps which are not protected by handrails or guardrails shall have flared sides with a slope not exceeding a ratio of 1 to 12.

FACBC 4.7.5 and Figure 12(b)

Draft 2012 FACBC section 406.3

(6)  Notwithstanding the requirements in reference 4.13.11 of the guidelines, exterior hinged doors shall be so designed that such doors can be pushed or pulled open with a force not exceeding 8.5 foot pounds.

FACBC 4.13.11(2)(a)

Draft 2012 FACBC section 404.2.9

(7)  Notwithstanding the requirements in reference 4.33.1 of the guidelines, all public food service establishments, all establishments licensed under the Beverage Law for consumption on the premises, and all facilities governed by reference 4.1 of the guidelines shall provide seating or spaces for seating in accordance with the following requirements:

FACBC 4.1.3 (19)

Draft 2012 FACBC section 226.1.1

(a)  For the first 100 fixed seats, accessible and usable spaces must be provided consistent with the following table:

| Capacity of Seating In Assembly Areas | Number of Required Wheelchair Locations |
| --- | --- |
| 1 to 25 ....... | 1 |
| 26 to 50 ....... | 2 |
| 51 to 100 ....... | 4 |

FACBC 4.1.3(19)(a)

Draft 2012 FACBC section 226.1.1

(b)  For all remaining fixed seats, there shall be not less than one such accessible and usable space for each 100 fixed seats or fraction thereof.

FACBC 4.1.3(19)(a)

Draft 2012 FACBC section 226.1.1

(8)  Notwithstanding the requirements in references 4.32.1-4.32.4 of the guidelines, all fixed seating in public food service establishments, in establishments licensed under the Beverage Law for consumption on the premises, and in all other facilities governed by reference 4.1 of the guidelines shall be designed and constructed in accordance with the following requirements:

(a)  All aisles adjacent to fixed seating shall provide clear space for wheelchairs.

(b)  Where there are open positions along both sides of such aisles, the aisles shall be not less than 52 inches wide.

FACBC 4.32.2

Draft 2012 FACBC section 206.2.5.1

(9)  In motels and hotels a number of rooms equaling at least 5 percent of the guest rooms minus the number of accessible rooms required by the guidelines shall provide the following special accessibility features:

(a)  Grab rails in bathrooms and toilet rooms that comply with s. 4.16.4 of the guidelines.

(b)  All beds in designed accessible guest rooms shall be open-frame type to permit passage of lift devices.

(c)  All standard water closet seats shall be at a height of 15 inches, measured vertically from the finished floor to the top of the seat, with a variation of plus or minus 1/2 inch. A portable or attached raised toilet seat shall be provided in all designated handicapped accessible rooms. 

All buildings, structures, or facilities licensed as a hotel, motel, or condominium pursuant to chapter 509 shall be subject to the provisions of this subsection. Nothing in this subsection shall be construed as relieving the owner of the responsibility of providing accessible rooms in conformance with ss. 9.1-9.5 of the guidelines.

FACBC 9.1.2 (Text shaded in yellow is combined and inserted) and FACBC 9.2.3 (Text shaded in yellow is combined and inserted as lead to (a), (b) and (c). Text shaded in blue is inserted ahead of the lead paragraph. Criteria shaded in gray are inserted after lead paragraph.)

Draft 2012 FACBC section 224.2 (Text shaded in yellow is combined and inserted and Table 224.2 modified for 5% criteria)

Draft 2012 FACBC section 806.4 (Text shaded in yellow is combined and inserted as lead to (a), (b) and (c). Text shaded in blue is inserted ahead of the lead paragraph. Criteria shaded in gray are inserted after lead paragraph.)

(10)  Notwithstanding the requirements in reference 4.29.2 of the guidelines, all detectable warning surfaces required by the guidelines shall be governed by the requirements of American National Standards Institute A117.1-1986.

FACBC 4.29.2(1)

Draft 2012 FACBC section 705.1

(11)  Notwithstanding the requirements in references 4.31.2 and 4.31.3 of the guidelines, the installation and placement of all public telephones shall be governed by the rules of the Florida Public Service Commission.

FACBC (Note there are no indications in the FACBC of changes to ADAAG)

Draft 2012 FACBC (Note if included in the 2012 code it should be in section 217)

(12)  Notwithstanding the requirements in references 4.1.3(11) and 4.16-4.23 of the guidelines, required restrooms and toilet rooms in new construction shall be designed and constructed in accordance with the following requirements:

(Note there is no indication in FACBC 4.1.3(11) of change to ADAAG)

Draft 2012 FACBC sections 213.2.2 and 604.8.1.6

(a)  The standard accessible restroom stall shall contain an accessible lavatory within it, the size of such lavatory to be not less than 19 inches wide by 17 inches deep, nominal size, and wall-mounted. The lavatory shall be mounted so as not to overlap the clear floor space areas required by s. 4.17 figure 30(a) of the guidelines for the standard accessible stall and to comply with s. 4.19 of the guidelines. Such lavatories shall be counted as part of the required fixture count for the building.

FACBC 4.17.3 added Exception (1)

Draft 2012 FACBC section 604.8.1.6.1

(b)  The accessible water closet shall be located in the corner, diagonal to the door.

FACBC 4.17.3 added Exception (2)

Draft 2012 FACBC section 604.8.1.6.2

(c)  The accessible stall door shall be self-closing.

FACBC 4.17.5 (Requirement imbedded in ADAAG text)

Draft 2012 FACBC section 604.8.1.6.3

(13)  All customer checkout aisles not required by the guidelines to be handicapped accessible shall have at least 32 inches of clear passage.

FACBC 7.3(4)

Draft 2012 FACBC section 904.3.4

(14)  Turnstiles shall not be used in occupancies which serve fewer than 100 persons, but turnstiles may be used in occupancies which serve at least 100 persons if there is an unlocked alternate passageway on an accessible route affording not less than 32 inches of clearance, equipped with latching devices in accordance with the guidelines.

FACBC 4.13.2(2)

Draft 2012 FACBC section 404.3.8

(15)  Barriers at common or emergency entrances and exits of business establishments conducting business with the general public that are existing, under construction, or under contract for construction which would prevent a person from using such entrances or exits shall be removed.

FACBC  4.1.8(2)

Draft 2012 FACBC section 202.1.1

History.--s. 1, ch. 93-183; s. 3, ch. 97-76.

553.5041  Parking spaces for persons who have disabilities.--

(1)  This section is not intended to expand or diminish the defenses available to a place of public accommodation under the Americans with Disabilities Act and the federal Americans with Disabilities Act Accessibility Guidelines, including, but not limited to, the readily achievable standard, and the standards applicable to alterations to places of public accommodation. Subject to the exceptions described in subsections (2), (4), (5), and (6), when the parking and loading zone requirements of the federal Americans with Disabilities Act Accessibility Guidelines (ADAAG), as adopted by reference in 28 C.F.R. part 36, subparts A and D, and Title II of Pub. L. No. 101-336, provide increased accessibility, those requirements are adopted and incorporated by reference as the law of this state.

FACBC  Not Found

Draft 2012 FACBC section 101.3

(2)  State agencies and political subdivisions having jurisdiction over street parking or publicly owned or operated parking facilities are not required to provide a greater right-of-way width than would otherwise be planned under regulations, guidelines, or practices normally applied to new development.

FACBC  4.1.2(5)(f)

Draft 2012 FACBC section 208.2.5.2

(3)  If parking spaces are provided for self-parking by employees or visitors, or both, accessible spaces shall be provided in each such parking area. Such spaces shall be designed and marked for the exclusive use of those individuals who have a severe physical disability and have permanent or temporary mobility problems that substantially impair their ability to ambulate and who have been issued either a disabled parking permit under s. 316.1958 or s. 320.0848 or a license plate under s. 320.084, s. 320.0842, s. 320.0843, or s. 320.0845.

FACBC  4.1.2(5)(a)

Draft 2012 FACBC section 208.3.1 (inserted with text from other subsections in law)

(4)  The number of accessible parking spaces must comply with the parking requirements in ADAAG s. 4.1 and the following:

(a)  There must be one accessible parking space in the immediate vicinity of a publicly owned or leased building that houses a governmental entity or a political subdivision, including, but not limited to, state office buildings and courthouses, if no parking for the public is provided on the premises of the building.

(b)  There must be one accessible parking space for each 150 metered on-street parking spaces provided by state agencies and political subdivisions.

(c)  The number of parking spaces for persons who have disabilities must be increased on the basis of demonstrated and documented need.

FACBC  4.12(5)(a) (i), (ii) and (iii)

Draft 2012 FACBC: Lead paragraph to (4) is not included; (a) is inserted in section 208.2.5.1; (b) is inserted in section 208.2.5.2; (c) is inserted in section 208.2.1

(5)  Accessible perpendicular and diagonal accessible parking spaces and loading zones must be designed and located in conformance with the guidelines set forth in ADAAG ss. 4.1.2 and 4.6 and Appendix s. A4.6.3 "Universal Parking Design."

FACBC  (See specifics below)

Draft 2012 FACBC – not in draft

(a)  All spaces must be located on an accessible route no less than 44 inches wide so that users will not be compelled to walk or wheel behind parked vehicles.

FACBC 4.3.3 and 4.6.2(1)

Draft 2012 FACBC section 208.3.1 (inserted together with other text from law)

(b)  Each space must be located on the shortest safely accessible route from the parking space to an accessible entrance. If there are multiple entrances or multiple retail stores, the parking spaces must be dispersed to provide parking at the nearest accessible entrance. If a theme park or an entertainment complex as defined in s. 509.013(9) provides parking in several lots or areas from which access to the theme park or entertainment complex is provided, a single lot or area may be designated for parking by persons who have disabilities, if the lot or area is located on the shortest safely accessible route to an accessible entrance to the theme park or entertainment complex or to transportation to such an accessible entrance.

FACBC 4.6.2 ADAAG language same as Florida law not shaded above               FACBC   ***** Can not locate text shaded in yellow in current code  *****                          FACBC 4.6.2(2) and 12(2)    (Text shaded in blue)

Draft 2012 FACBC section 208.3.1 (Text shaded in yellow)                                  Draft 2012 FACBC section 208.3.1 Exception 3. (Text shaded in blue)

(c)1.  Each parking space must be no less than 12 feet wide. Parking access aisles must be no less than 5 feet wide and must be part of an accessible route to the building or facility entrance. In accordance with ADAAG s. 4.6.3, access aisles must be placed adjacent to accessible parking spaces; however, two accessible parking spaces may share a common access aisle. The access aisle must be striped diagonally to designate it as a no-parking zone.

FACBC 4.6.1 Text shaded in yellow on parking space width                     FACBC 4.6.3 All text shaded in blue on the access aisle inserted.

Draft 2012 FACBC section: Text on space width inserted in 502.2

Text shaded yellow on access aisle on accessible route to entrance inserted in 502.3

Text on access aisle width was not added to the draft code (defer to ADAAG same requirement)

Text on access aisle adjacent to accessible parking space inserted in 502.3

Text on accessible parking spaces sharing access aisle was not added to draft code (defer to ADAAG same provision)

Text on access aisle striped diagonally inserted in 502.3.3  Note that ADAAG section 502.2 contains similar provision

2.  The parking access aisles are reserved for the temporary exclusive use of persons who have disabled parking permits and who require extra space to deploy a mobility device, lift, or ramp in order to exit from or enter a vehicle. Parking is not allowed in an access aisle. Violators are subject to the same penalties that are imposed for illegally parking in parking spaces that are designated for persons who have disabilities. A vehicle may not be parked in an access aisle, even if the vehicle owner or passenger is disabled or owns a disabled parking permit.

Not in FACBC -  Not a building construction requirement

Not in Draft 2012 FACBC

3.  Any provision of this subsection to the contrary notwithstanding, a theme park or an entertainment complex as defined in s. 509.013(9) in which are provided continuous attendant services for directing individuals to marked accessible parking spaces or designated lots for parking by persons who have disabilities, may, in lieu of the required parking space design, provide parking spaces that comply with ADAAG ss. 4.1 and 4.6.

FACBC 4.6.3 Exception includes text shaded yellow and section 12(1) entire text of 3. is inserted together with other text from law.

Draft 2012 FACBC section 208.3.1 Exception 4 includes the entire text of 3.

(d)  On-street parallel parking spaces must be located either at the beginning or end of a block or adjacent to alley entrances. Such spaces must be designed in conformance with the guidelines set forth in ADAAG ss. 4.6.2 through 4.6.5, exception: access aisles are not required. Curbs adjacent to such spaces must be of a height that will not interfere with the opening and closing of motor vehicle doors. This subsection does not relieve the owner of the responsibility to comply with the parking requirements of ADAAG ss. 4.1 and 4.6.

FACBC  4.6.2(3)    (Note- Yellow shaded text is in this section of the Code) FACBC 4.6.3    (Note- Sky and aqua blue shaded text is in this section of the Code. See note on (e).)

Draft 2012 FACBC section: Text shaded yellow on location inserted in 502.2.2

Text unshaded on designed in conformance with ADAAG not in draft

Text shaded sky blue on access aisle not required inserted as exception to 502.3 (language clarifies exception applies to on-street parallel parking unlike current code)

Text shaded aqua blue on curbs inserted in 502.2

Text unshaded on not relieving owner of responsibility to comply with ADAAG not in draft

(e)  Parallel parking spaces must be even with surface slopes, may match the grade of the adjacent travel lane, and must not exceed a cross slope of 1 to 50, where feasible.

FACBC 4.6.3 (Note: Language from (e) combined with exception on access aisles and curb height requirements in (d). The (d) exception and curb height requirement are separated from the “on-street parallel parking which is located in 4.6.2(3).)

Draft 2012 FACBC section 502.2.1

(f)  Curb ramps must be located outside of the disabled parking spaces and access aisles.

FACBC 4.7.1

Draft 2012 FACBC section 406.5 (Considered including in this section but the ADAAG language establishes the same requirement. Consider removing from law)

(g)1.  The removal of architectural barriers from a parking facility in accordance with 28 C.F.R. s. 36.304 or with s. 553.508 must comply with this section unless compliance would cause the barrier removal not to be readily achievable. If compliance would cause the barrier removal not to be readily achievable, a facility may provide parking spaces at alternative locations for persons who have disabilities and provide appropriate signage directing persons who have disabilities to the alternative parking if readily achievable. The facility may not reduce the required number or dimensions of those spaces, nor may it unreasonably increase the length of the accessible route from a parking space to the facility. The removal of an architectural barrier must not create a significant risk to the health or safety of a person who has a disability or to that of others.

FACBC 4.1.8(3)

Draft 2012 FACBC section 208.1.1

2.  A facility that is making alterations under s. 553.507(2)(b) must comply with this section to the maximum extent feasible. If compliance with parking location requirements is not feasible, the facility may provide parking spaces at alternative locations for persons who have disabilities and provide appropriate signage directing persons who have a disability to alternative parking. The facility may not reduce the required number or dimensions of those spaces, nor may it unnecessarily increase the length of the accessible route from a parking space to the facility. The alteration must not create a significant risk to the health or safety of a person who has a disability or to that of others.

FACBC  4.1.6(k)(iv) Just the text shaded gray.

Draft 2012 FACBC section 208.1.2 Text shaded gray and text shaded yellow.

(6)  Each such parking space must be prominently outlined with blue paint, and must be repainted when necessary, to be clearly distinguishable as a parking space designated for persons who have disabilities and must be posted with a permanent above-grade sign of a color and design approved by the Department of Transportation, which is placed on or at a distance of 84 inches above the ground to the bottom of the sign and which bears the international symbol of accessibility meeting the requirements of ADAAG s. 4.30.7 and the caption "PARKING BY DISABLED PERMIT ONLY." Such a sign erected after October 1, 1996, must indicate the penalty for illegal use of the space. Any provision of this section to the contrary notwithstanding, in a theme park or an entertainment complex as defined in s. 509.013(9) in which accessible parking is located in designated lots or areas, the signage indicating the lot as reserved for accessible parking may be located at the entrances to the lot in lieu of a sign at each parking place. This subsection does not relieve the owner of the responsibility of complying with the signage requirements of ADAAG s. 4.30.

FACBC 4.6.4- Text shaded in yellow is inserted in first and third paragraphs.

Draft 2012 FACBC section 502.6.2  Text shaded in yellow and in blue are included.

History.--s. 66, ch. 2000-141.

553.505  Exceptions to applicability of the Americans with Disabilities Act.--Notwithstanding the Americans with Disabilities Act of 1990, private clubs are governed by ss. 553.501-553.513. Parking spaces, parking lots, and other parking facilities are governed by s. 553.5041 when that section provides increased accessibility.

History.--s. 1, ch. 93-183; s. 14, ch. 96-200; s. 4, ch. 97-76; s. 23, ch. 2001-186.

FACBC  1.3

Draft 2012 FACBC section 101.1.3

553.506  Powers of the commission.--In addition to any other authority vested in the Florida Building Commission by law, the commission, in implementing ss. 553.501-553.513, may, by rule, adopt revised and updated versions of the Americans with Disabilities Act Accessibility Guidelines in accordance with chapter 120.

History.--s. 1, ch. 93-183; s. 67, ch. 2000-141; s. 60, ch. 2000-154.

NOT A PART OF FACBC

553.507  Exemptions.--Sections 553.501-553.513 do not apply to any of the following:

(1)  Buildings, structures, or facilities that were either under construction or under contract for construction on October 1, 1997.

FACBC  4.1.1(5)(b) and 4.1.2 and 4.1.3

Draft 2012 FACBC section 201.1 Exception (1)

(2)  Buildings, structures, or facilities that were in existence on October 1, 1997, unless:

(a)  The building, structure, or facility is being converted from residential to nonresidential or mixed use, as defined by local law;

(b)  The proposed alteration or renovation of the building, structure, or facility will affect usability or accessibility to a degree that invokes the requirements of s. 303(a) of the Americans with Disabilities Act of 1990; or

(c)  The original construction or any former alteration or renovation of the building, structure, or facility was carried out in violation of applicable permitting law.

FACBC 4.1.6 Text shaded yellow and text shaded blue.                           FACBC 11.1(2)(a) Text shaded in blue.

Draft 2012 FACBC section 201.1 Exception (2) Text shaded in yellow and text shaded in blue.

History.--s. 1, ch. 93-183; s. 5, ch. 97-76; s. 31, ch. 2001-63; s. 24, ch. 2001-186.

553.508  Architectural barrier removal.--Removal of architectural barriers, pursuant to 28 C.F.R. s. 36.304, from buildings, structures, or facilities to which this act applies shall comply with ss. 553.501-553.513 unless compliance would render the removal not readily achievable. In no instance shall the removal of an architectural barrier create a significant risk to the health or safety of an individual with a disability or others.

FACBC 4.1.1(1)(b) and 4.1.8(1)

Draft 2012 FACBC section 202.1.2

History.--s. 1, ch. 93-183.

553.509  Vertical accessibility.--

(1)  Nothing in ss. 553.501-553.513 or the guidelines shall be construed to relieve the owner of any building, structure, or facility governed by those sections from the duty to provide vertical accessibility to all levels above and below the occupiable grade level, regardless of whether the guidelines require an elevator to be installed in such building, structure, or facility, except for:

(a)  Elevator pits, elevator penthouses, mechanical rooms, piping or equipment catwalks, and automobile lubrication and maintenance pits and platforms;

(b)  Unoccupiable spaces, such as rooms, enclosed spaces, and storage spaces that are not designed for human occupancy, for public accommodations, or for work areas; and

(c)  Occupiable spaces and rooms that are not open to the public and that house no more than five persons, including, but not limited to, equipment control rooms and projection booths.

FACBC 4.1.2 and 4.1.3(5) and 4.1.6(1)(f) and 4.1.6(1)(k)(iii) and 5.4 Exception

Draft 2012 FACBC section 201.1.1                                                            Section 201.1.1 referred to from sections 202.3, 202.4, 203.1, 206.2.3, 206.2.4, 206.2.5, 206.2.7, 221.1, 240.1, 240.2.1.1, 240.2.1.2, 240.2.2, 241,

(2)(a)  Any person, firm, or corporation that owns, manages, or operates a residential multifamily dwelling, including a condominium, that is at least 75 feet high and contains a public elevator, as described in s. 399.035(2) and (3) and rules adopted by the Florida Building Commission, shall have at least one public elevator that is capable of operating on an alternate power source for emergency purposes. Alternate power shall be available for the purpose of allowing all residents access for a specified number of hours each day over a 5-day period following a natural disaster, manmade disaster, emergency, or other civil disturbance that disrupts the normal supply of electricity. The alternate power source that controls elevator operations must also be capable of powering any connected fire alarm system in the building.

NOT A PART OF FACBC

(b)  At a minimum, the elevator must be appropriately prewired and prepared to accept an alternate power source and must have a connection on the line side of the main disconnect, pursuant to National Electric Code Handbook, Article 700. In addition to the required power source for the elevator and connected fire alarm system in the building, the alternate power supply must be sufficient to provide emergency lighting to the interior lobbies, hallways, and other portions of the building used by the public. Residential multifamily dwellings must have an available generator and fuel source on the property or have proof of a current contract posted in the elevator machine room or other place conspicuous to the elevator inspector affirming a current guaranteed service contract for such equipment and fuel source to operate the elevator on an on-call basis within 24 hours after a request. By December 31, 2006, any person, firm or corporation that owns, manages, or operates a residential multifamily dwelling as defined in paragraph (a) must provide to the local building inspection agency verification of engineering plans for residential multifamily dwellings that provide for the capability to generate power by alternate means. Compliance with installation requirements and operational capability requirements must be verified by local building inspectors and reported to the county emergency management agency by December 31, 2007.

NOT A PART OF FACBC

(c)  Each newly constructed residential multifamily dwelling, including a condominium, that is at least 75 feet high and contains a public elevator, as described in s. 399.035(2) and (3) and rules adopted by the Florida Building Commission, must have at least one public elevator that is capable of operating on an alternate power source for the purpose of allowing all residents access for a specified number of hours each day over a 5-day period following a natural disaster, manmade disaster, emergency, or other civil disturbance that disrupts the normal supply of electricity. The alternate power source that controls elevator operations must be capable of powering any connected fire alarm system in the building. In addition to the required power source for the elevator and connected fire alarm system, the alternate power supply must be sufficient to provide emergency lighting to the interior lobbies, hallways, and other portions of the building used by the public. Engineering plans and verification of operational capability must be provided by the local building inspector to the county emergency management agency before occupancy of the newly constructed building.

NOT A PART OF FACBC

(d)  Each person, firm, or corporation that is required to maintain an alternate power source under this subsection shall maintain a written emergency operations plan that details the sequence of operations before, during, and after a natural or manmade disaster or other emergency situation. The plan must include, at a minimum, a lifesafety plan for evacuation, maintenance of the electrical and lighting supply, and provisions for the health, safety, and welfare of the residents. In addition, the owner, manager, or operator of the residential multifamily dwelling must keep written records of any contracts for alternative power generation equipment. Also, quarterly inspection records of lifesafety equipment and alternate power generation equipment must be posted in the elevator machine room or other place conspicuous to the elevator inspector, which confirm that such equipment is properly maintained and in good working condition, and copies of contracts for alternate power generation equipment shall be maintained on site for verification. The written emergency operations plan and inspection records shall also be open for periodic inspection by local and state government agencies as deemed necessary. The owner or operator must keep a generator key in a lockbox posted at or near any installed generator unit.

NOT A PART OF FACBC

(e)  Multistory affordable residential dwellings for persons age 62 and older that are financed or insured by the United States Department of Housing and Urban Development must make every effort to obtain grant funding from the Federal Government or the Florida Housing Finance Corporation to comply with this subsection. If an owner of such a residential dwelling cannot comply with the requirements of this subsection, the owner must develop a plan with the local emergency management agency to ensure that residents are evacuated to a place of safety in the event of a power outage resulting from a natural or manmade disaster or other emergency situation that disrupts the normal supply of electricity for an extended period of time. A place of safety may include, but is not limited to, relocation to an alternative site within the building or evacuation to a local shelter.

NOT A PART OF FACBC

(f)  As a part of the annual elevator inspection required under s. 399.061, certified elevator inspectors shall confirm that all installed generators required by this chapter are in working order, have current inspection records posted in the elevator machine room or other place conspicuous to the elevator inspector, and that the required generator key is present in the lockbox posted at or near the installed generator. If a building does not have an installed generator, the inspector shall confirm that the appropriate prewiring and switching capabilities are present and that a statement is posted in the elevator machine room or other place conspicuous to the elevator inspector affirming a current guaranteed contract exists for contingent services for alternate power is current for the operating period. 

However, buildings, structures, and facilities must, as a minimum, comply with the requirements in the Americans with Disabilities Act Accessibility Guidelines.

NOT A PART OF FACBC

History.--s. 1, ch. 93-183; s. 6, ch. 97-76; s. 12, ch. 2006-71.

553.511  Parking facilities; minimum height clearance requirement.--Every nonresidential structure built on or after January 1, 1991, which is designed to use covered or underground parking as the primary available parking space shall design the covered or underground parking facility to maintain a minimum height for the portion of the street-accessible level of the parking facility directly over van-accessible parking spaces and for providing ingress and egress to such parking spaces of at least 8 feet 2 inches. Signs shall be posted to warn operators of handicapped-equipped vans that they cannot pass beyond a certain point due to height limitations. If compliance with this minimum height clearance requirement will cause the structure to exceed local height limitations imposed by local zoning, planning, or fire ordinances, or will result in the imposition of any additional requirements of such ordinances, the structure may exceed the height limitation specified in those particular codes as necessary to comply with the requirements of this section and is exempt from such additional requirements. Structures for which the plans were sealed by an architect prior to January 1, 1991, are exempt from this section.

History.--s. 2, ch. 90-250; s. 2, ch. 93-183; s. 7, ch. 97-76.

Note.--Former s. 553.482.

FACBC 4.6.5

Draft 2012 FACBC section 502.5

553.512  Modifications and waivers; advisory council.--

(1)  The Florida Building Commission shall provide by regulation criteria for granting individual modifications of, or exceptions from, the literal requirements of this part upon a determination of unnecessary, unreasonable, or extreme hardship, provided such waivers shall not violate federal accessibility laws and regulations and shall be reviewed by the Accessibility Advisory Council. Notwithstanding any other provision of this subsection, if an applicant for a waiver demonstrates economic hardship in accordance with 28 C.F.R. s. 36.403(f)(1), a waiver shall be granted. The commission may not consider waiving any of the requirements of s. 553.5041 unless the applicant first demonstrates that she or he has applied for and been denied waiver or variance from all local government zoning, subdivision regulations, or other ordinances that prevent compliance therewith. Further, the commission may not waive the requirement of s. 553.5041(5)(a) and (c)1. governing the minimum width of accessible routes and minimum width of accessible parking spaces.

NOT A PART OF FACBC

(2)  The Accessibility Advisory Council shall consist of the following seven members, who shall be knowledgeable in the area of accessibility for persons with disabilities. The Secretary of Community Affairs shall appoint the following: a representative from the Advocacy Center for Persons with Disabilities, Inc.; a representative from the Division of Blind Services; a representative from the Division of Vocational Rehabilitation; a representative from a statewide organization representing the physically handicapped; a representative from the hearing impaired; a representative from the President, Florida Council of Handicapped Organizations; and a representative of the Paralyzed Veterans of America. The terms for the first three council members appointed subsequent to October 1, 1991, shall be for 4 years, the terms for the next two council members appointed shall be for 3 years, and the terms for the next two members shall be for 2 years. Thereafter, all council member appointments shall be for terms of 4 years. No council member shall serve more than two 4-year terms subsequent to October 1, 1991. Any member of the council may be replaced by the secretary upon three unexcused absences. Upon application made in the form provided, an individual waiver or modification may be granted by the commission so long as such modification or waiver is not in conflict with more stringent standards provided in another chapter.

NOT A PART OF FACBC

(3)  Members of the council shall serve without compensation, but shall be entitled to reimbursement for per diem and travel expenses as provided by s. 112.061.

NOT A PART OF FACBC

(4)  Meetings of the advisory council shall be held in conjunction with the regular meetings of the commission.

NOT A PART OF FACBC

History.--s. 3, ch. 78-333; s. 1, ch. 82-46; s. 2, ch. 83-265; s. 25, ch. 86-220; s. 5, ch. 89-97; ss. 1, 5, 6, ch. 91-172; s. 5, ch. 91-429; s. 2, ch. 93-183; s. 10, ch. 97-76; s. 68, ch. 2000-141; s. 61, ch. 2000-154; s. 13, ch. 2002-293.

Note.--Former s. 553.49.

553.513  Enforcement.--It shall be the responsibility of each local government and each code enforcement agency established pursuant to s. 553.80 to enforce the provisions of this part. This act expressly preempts the establishment of handicapped accessibility standards to the state and supersedes any county or municipal ordinance on the subject. However, nothing in this section shall prohibit municipalities and counties from enforcing the provisions of this act.

History.--s. 6, ch. 89-97; s. 2, ch. 93-183.

Note.--Former s. 553.495.

NOT A PART OF FACBC

Explanation of conventions:

FACBC means current Florida Accessibility Code and identifies sections text from law is located.

Draft 2012 FACBC section identifies the location of text from law in draft code.

All text actually transferred into the FACBC and Draft 2012 FACBC are identified in gray, yellow or blue (two shades in one place) to designate which sections the highlighted text is place in.